| 02/18/00 | Introduced by: _____________________ |
| --- | --- |
| (PK/SB/KA/DK) | Proposed No.: _____________________ |

- ..title

- AN ORDINANCE relating to zoning and comprehensive planning, revising definitions of the rural area zone and urban residential zone to be consistent with comprehensive plan policies, adding definitions for hazardous materials pipelines, mixed-use development, rural forest focus areas and resource land tract, amending permitted use tables to address development standards and allowed uses in the rural area and on resource lands and apartments and townhouses in the R-1 through R-8 zones, amending density and dimensions tables to add a minimum lot area to the rural area zone and allowing a density of one dwelling unit per five acres for density transfers, clarifying the minimum density requirements to recognize that P-Suffix or Special District overlay may alter minimum density requirements, revising the method for calculating density in the rural area, establishing setbacks from hazardous materials pipelines, restricting uses within a hazardous materials pipeline easement, allowing limited movement of density within a development site consisting of lots with different zoning, amending and creating new development standards relating to size of buildings, setbacks, building placement and landscaping for non-residential uses in the rural area, amending standards for cluster development in the rural area to allow creation of a tract for resource uses and requiring native landscaping along frontage of new cluster subdivisions, establishing site design standards and construction timing for mixed-use developments, amending on-site recreation space requirements and clarifying play area requirements, changing the calculation for fee-in-lieu of recreation space to reflect market value after development, increasing landscaping of surface parking lots and requirements for street trees, strengthening requirements to place parking to the rear or side of buildings in non-residential developments, restricting wetland mitigation banking sites from the Agricultural Production District, clarifying standards for caretaker’s accessory living quarters when located within a barn or stable, adding density incentives for providing public art in private development projects, requiring the review process for new high schools in the urban area to apply to new high schools in the rural area, changing the term “rural forest district” to “rural forest focus areas”; amending Ordinance 10870, Section 27, as amended, and K.C.C. 21A.04.060, Ordinance 10870, Section 29, as amended, and K.C.C. 21A.04.080, Ordinance 10870, Section 330, as amended, and K.C.C. 21A.08.030, Ordinance 10870, Section 331, as amended, and K.C.C. 21A.08.040, Ordinance 10870, Section 332, as amended, and K.C.C. 21A.08.050, Ordinance 10870, Section 334, as amended, and K.C.C. 21A.08.070, Ordinance 10870, Section 335, as amended, and K.C.C. 21A.08.080, Ordinance 10870, Section 336, as amended, and K.C.C. 21A.08.090, Ordinance 10870, Section 337, as amended, and K.C.C. 21A.08.100, Ordinance 10870, Section 340, as amended, and K.C.C. 21A.12.030, Ordinance 10870, Section 341, as amended, and K.C.C. 21A.12.040, Ordinance 10870, Section 343, as amended, and K.C.C. 21A.12.060, Ordinance 10870, Section 344, as amended, and K.C.C. 21A.12.070, Ordinance 10870, Section 345, as amended, and K.C.C. 21A.12.080, Ordinance 10870, Section 351, as amended, and K.C.C. 21A.12.140, Ordinance 10870, Section 357, as amended, and K.C.C. 21A.12.200, Ordinance 10870, Section 359, as amended, and K.C.C. 21A.12.220, Ordinance 10870, Section 364, as amended, and K.C.C. 21A.14.040, Ordinance 10870, Section 378, as amended, and K.C.C. 21A.14.180, Ordinance 11621, Section 49, and K.C.C. 21A.14.185, Ordinance 10870, Section 379, as amended, and K.C.C. 21A.14.190, Ordinance 10870, Section 380,as amended, and K.C.C. 21A.14.200, Ordinance 10870, Section 388, as amended, and K.C.C. 21A.16.030, Ordinance 10870, Section 390, as amended, and K.C.C. 21A.16.050, Ordinance 10870, Section 392, as amended, and K.C.C. 21A.16.070, Ordinance 10870, Section 415, as amended, and K.C.C. 21A.18.110, Ordinance 10870, Section 481, as amended, and K.C.C. 21A.24.340, Ordinance 11621, Section 72, and K.C.C. 21A.24.345, Ordinance 11168, Section 5, as amended, and K.C.C. 21A.30.062, Ordinance 10870, Section 563, and K.C.C. 21A.34.040, Ordinance 10870, Section 634 (part), and K.C.C. 21A.42.140, Ordinance 13274, Section 2, and K.C.C. 21A.55.110, Ordinance 13274, Section 4, and K.C.C. 21A.55.130, Ordinance 13274, Section 5, and K.C.C. 21A.55.140, Ordinance 13274, Section 6, and K.C.C. 21A.55.150, adding new sections to K.C.C. chapter 21A.06, adding a new section to K.C.C. chapter 21A.12, adding new sections to K.C.C. chapter 21A.14 and repealing Ordinance 10870, Section 374, and K.C.C. 21A.14.140 and Ordinance 12823, Section 19, and K.C.C. 21A.38.240.

..body

- PREAMBLE:

- For the purposes of effective land use planning and regulation, the King County Council makes the following legislative findings:

- King County has adopted the 1994 King County Comprehensive Plan to meet the requirements of the Washington State Growth Management Act (GMA).

- The GMA requires that the Comprehensive Plan and development regulations be subject to continuing review and evaluation by the county.

- King County has approved annual amendments to correct technical errors and to make changes that do not require a substantive policy change or alter the urban growth line.

- King County has performed its first comprehensive four-year cycle review of the Comprehensive Plan and development regulations.   As a result of the review, King County is amending the 1994 Comprehensive through passage of the 2000 King County Comprehensive Plan.

- The GMA requires that King County adopt development regulations, to be consistent with and implement the Comprehensive Plan.

- The changes to the King County zoning code, K.C.C. Title 21A, contained in this ordinance are needed to bring K.C.C. Title 21A into conformance with the 2000 King County Comprehensive Plan, as required by the GMA.  As such they bear a substantial relationship to, and are necessary for, the public health, safety and general welfare of King County and its residents.

- BE IT ORDAINED BY THE COUNCIL OF KING COUNTY:

SECTION 1.  Ordinance 10870, Section 27, as amended, and K.C.C. 21A.04.060 are each hereby amended to read as follows:

Rural area zone.  A.  The purpose of the rural zone (RA) is to provide for an area-wide long-term rural character and to minimize land use conflicts with nearby agricultural((,)) or forest ((or mineral extraction)) production districts or mineral extraction sites.  These purposes are accomplished by:

1.  Limiting residential densities and permitted uses to those that are compatible with rural character and nearby resource production districts and sites and are able to be adequately supported by rural service levels;

2.  Allowing small scale farming and forestry activities and tourism and recreation uses which can be supported by rural service levels and which are compatible with rural character; ((and))

3.  Increasing required setbacks to minimize conflicts with adjacent agriculture, forest or mineral zones((.)); and

4.  Requiring tracts created through cluster development to be designated as permanent open space or as permanent resource use.

B.  Use of this zone is appropriate in rural areas designated by the Comprehensive Plan as follows:

1.  ((AR-2.5/))RA-2.5 in rural areas where the predominant ((densities already exceed one dwelling per)) lot pattern is below five acres in size for lots established prior to the adoption of the 1994 Comprehensive Plan ((and the soils can support on-site sewage disposal without damage to water resources));

2.  ((AR-5/))RA-5 in rural areas ((without established subdivision patterns and predominantly)) where the predominant lot pattern is five acres or greater but less than ten acres in size and the area is generally environmentally unconstrained ((lands));

3.  ((AR-10/))RA-10 in rural areas where the predominant lot pattern is ten acres or greater but less than twenty acres in size.  RA-10 is also applied on land that is generally environmentally constrained, as defined by county, state or federal law; to protect critical habitat; to protect regionally significant resource areas (RSRAs); to provide a transition ((next)) to ((designated resource production areas)) the forest production district and agricultural production districts; and to buffer mineral extraction sites where additional buffering is required ((, or where area-wide environmental features constrain development. RA-10 is also applied to Rural Farm Districts designated by the King County Comprehensive Plan)) On Vashon-Maury Island RA-10 zoning shall be maintained on areas zoned RA-10 as of 1994 and on areas with a predominant lot size of ten acres or greater that are identified on the Areas Highly Susceptible to Groundwater Contamination map; and  [Editor's Note:  Ordinance 11621 § 10 did not delete `AR' in 1-3 when `RA' was added.]

4.  RA-20 where the predominant lot pattern is twenty acres or greater.  RA-20 is also applied to provide a transition to the ((in Rural F))forest production ((D))district((s)) and agricultural production districts designated by the King County Comprehensive Plan and to protect critical habitat areas.

C.  Lots in the RA zone can be developed provided:

1.  Soils can support on-site sewage disposal without damage to water resources;

2.  Water can be provided in accordance with county, state and federal law;

3.  Roads meet county concurrency and road standards; and

4.  The development can meet standards for rural fire protection.

SECTION 2.  Ordinance 10870, Section 29, as amended, and K.C.C. 21A.04.080 are each hereby amended to read as follows:

Urban residential zone.  A.  The purpose of the urban residential zone (R) is to implement comprehensive plan goals and policies for housing quality, diversity and affordability, and to efficiently use urban residential land, public services and energy.  These purposes are accomplished by:

1.  Providing, in the R-1 through R-8 zones, for a mix of predominantly single detached dwelling units and other development types, with a variety of densities and sizes in locations appropriate for urban densities;

2.  Providing, in the R-12 through R-48 zones, for a mix of predominantly apartment and townhouse dwelling units, mixed use and other development types, with a variety of densities and sizes in locations appropriate for urban densities;

3.  Allowing only those accessory and complementary nonresidential uses that are compatible with urban residential communities; and

4.  Establishing density designations to facilitate advanced area-wide planning for public facilities and services, and to protect environmentally sensitive sites from overdevelopment.

B.  Use of this zone is appropriate in urban areas, activity centers, or ((r))Rural ((t))Towns designated by the comprehensive plan ((or community plans)) as follows:

1.  The R-1 zone on or adjacent to lands with area-wide environmental constraints where development is required to cluster away from sensitive areas, on lands designated urban separators or wildlife habitat network where development is required to cluster away from the axis of the corridor((,)) on critical aquifer recharge areas, and on Regionally and Locally Significant Resource Areas (RSRAs/LSRAs) ((or in well-established subdivisions of the same density, which are served at the time of development by public or private facilities and services adequate to support planned densities));

2.  The R-4 through R-8 zones on urban lands that are predominantly environmentally unconstrained and are served at the time of development, by adequate public sewers, water supply, roads and other needed public facilities and services; and

3.  The R-12 through R-48 zones in ((urban areas, urban or community)) or next to Unincorporated ((a))Activity ((c))Centers((, urban neighborhood centers,)); adjacent to Community or Neighborhood Business Centers; in mixed use developments; on small, scattered lots integrated into existing residential areas; or in ((r))Rural ((t))Towns, that are served at the time of development by adequate public sewers, water supply, roads and other needed public facilities and services.

NEW SECTION.  SECTION 3.  There is hereby added to K.C.C. chapter 21A.06 a new section to read as follows:

Hazardous materials pipeline.  Hazardous materials pipeline:  A.  Pipe with a nominal diameter of six inches or more used to transport hazardous liquid, but not including pipe used to transport hazardous liquid by gravity or pipe used to transport or store hazardous liquid within a refinery, storage or manufacturing facility.

B.  Pipe operated at a pressure of more than two-hundred-seventy-five pounds per square inch used to transport gas, but not including pipe used to transport or store gas within a refinery, storage or manufacturing facility.

NEW SECTION.  SECTION 4.  There is hereby added to K.C.C. chapter 21A.06 a new section to read as follows:

Mixed-use development.  Mixed-use development: a combination of residential and non-residential uses within the same building or site as part of an integrated development project with functional interrelationships and coherent physical design.

NEW SECTION.  SECTION 5.  There is hereby added to K.C.C. chapter 21A.06 a new section to read as follows:

Open space.  Open space: For purposes of this Title, open space means areas left predominately in a natural state to create urban separators and greenbelts, sustain native ecosystems, connect and increase protective buffers for environmentally sensitive areas, provide a visual contrast to continuous development, reinforce community identity and aesthetics, or provide links between important environmental or recreational resources.

NEW SECTION.  SECTION 6.  There is hereby added to K.C.C. chapter 21A.06 a new section to read as follows:

Rural forest focus areas.  Rural forest focus areas: Mapped geographic areas where special efforts to maintain forest cover and the practice of sustainable forestry are warranted.

NEW SECTION.  SECTION 7.  There is hereby added to K.C.C. chapter 21A.06 a new section to read as follows:

Resource land tract.  Resource land tract: For purposes of this Title, a tract of land, created through a subdivision or short subdivision cluster development in the RA zone, which may be used as a working forest or farm.

SECTION 8.  Ordinance 10870, Section 330, as amended, and K.C.C. 21A.08.030 are each hereby amended to read as follows:

A.  Residential land uses.

| KEY P- Permitted Use C- Conditional Use S- Special Use |  | Resource |  |  | Residential |  |  |  | Commercial/Industrial |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ###### A G R I C U L T U R E | #### F O R E S T | M I N E R A L | R U R A L | #### U R B A N | R E S E R V E | U R B A N | R E S I D E N T I A L | N E I G H B O R H O O D | B U S I N E S S | C O M M U N I T Y | B U S I N E S S | R E G I O N A L | B U S I N E S S | O F F I C E | I N D U S T R I A L |
| ##### Zone |  | A | F | M | RA | UR |  | R1-8 | R12-48 | NB |  | CB |  | RB |  | O | I |
| Specific Land Use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### SIC # | DWELLING UNITS, TYPES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * | Single detached | P((2)) C13 | P2 C13 |  | P((2)) C13 | P C13 |  | P C13 | P C13 |  |  |  |  |  |  |  |  |
| * | Townhouse |  |  |  |  |  |  | P ((12) C 12 | P | P3 |  | P3 |  | P3 |  | P3 |  |
| * | Apartment |  |  |  | C4 | C4 |  | P5 C4 | P | P3 |  | P3 |  | P3 |  | P3 |  |
| * | Mobile home park |  |  |  | S14 |  |  | C8 | P |  |  |  |  |  |  |  |  |
|  | GROUP RESIDENCES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * | Community residential facility - I |  |  |  | C | C |  | C | P | P3 |  | P3 |  | P3 |  | P3 |  |
| * | Community residential facility - II |  |  |  |  |  |  |  | P | P3 |  | P3 |  | P3 |  | P3 |  |
| * | Dormitory |  |  |  | C6 | C6 |  | C6 | P |  |  |  |  |  |  |  |  |
| * | Senior citizen assisted housing |  |  |  |  | P4 |  | P4 | P | P3 |  | P3 |  | P3 |  | P3 |  |
|  | ACCESSORY USES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * | Residential accessory uses | P7 | P7 |  | P7 | P7 |  | P7 | P7 | P7 |  | P7 |  | P7 |  | P7 |  |
| * | Home occupation | P | P |  | P | P |  | P | P | P |  | P |  | P |  | P |  |
| * | Home industry | C |  |  | C | C |  | C |  |  |  |  |  |  |  |  |  |
|  | TEMPORARY LODGING: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7011 | hotel/motel (1) |  |  |  |  |  |  |  |  |  |  | P |  | P |  | P |  |
| * | Bed and breakfast guesthouse | P9 C10 |  |  | P10 | P10 |  | P10 | P10 | P10 |  | P11 |  | P11 |  |  |  |
| 7041 | Organization hotel/lodging houses |  |  |  |  |  |  |  |  |  |  |  |  | P |  |  |  |
| GENERAL CROSS REFERENCES: Land Use Table Instructions, see K.C.C. 21A.08.020 and 21A.02.070; Development Standards, see K.C.C. 21A.12 through 21A.30; General Provisions, see K.C.C. 21A.32 through 21A.38 Application and Review Procedures, see K.C.C. 21A.40 through 21A.44; (*) Definition of this specific land use, see K.C.C. 21A.06. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- B.  Development conditions.

1.  Except bed and breakfast guesthouses.

2.  In the forest production district, the following conditions apply:

a.  A single detached dwelling unit is allowed only on a lot that was legally created on or before January 1, 2000.  A single detached dwelling unit is not allowed on any lot created after January 1, 2000.

b.  Only one single detached dwelling unit is allowed on a lot created on or before January 1, 2000, regardless of the size of the lot.

c.  Site disturbance associated with development of any new residence shall be limited to three acres for building envelope, accessory buildings and yard.  Additional site disturbance for raising livestock or growing crops may be approved as part of a farm management (conservation) plan developed by the King Conservation District.  If livestock management is proposed, the plan must meet the criteria in K.C.C. chapter 21A.30.

d.  A forest management plan shall be required for any new residence in the forest production district, which shall be reviewed and approved by the King County department of natural resources prior to building permit issuance.

e.  ((For properties in the F zone, a)) A fire protection plan for the subject property is required and shall be reviewed and approved by the Washington ((D))department of ((N))natural ((R))resources with the concurrence of the fire marshal for each residential use. This plan shall be developed in such a manner as to protect the adjoining forestry uses from a fire which may originate from the residential use.  This plan shall provide for setbacks from existing forestry uses and maintenance of approved fire trails or other effective fire line buffers on perimeters with forest land.

3.  Only as part of a mixed use development subject to the conditions of K.C.C. chapter 21A.14, except that in the NB zone on properties with a land use designation of commercial outside of center (CO) in the urban areas, stand-alone townhouse developments are permitted subject to the provisions of K.C.C. 21A.12.040, K.C.C. 21A.14.030, K.C.C. 21A.14.060 and K.C.C. 21A.14.180.

4.  ((Only in a building listed on the National Register as an historic site or designated as a King County landmark subject to the provisions of K.C.C. 21A.32))

Apartment units are permitted subject to a conditional use permit as follows:

a.  In the R-1 zone when exceeding base density and when at least fifty percent of the site is constrained by unbuildable sensitive areas.  For purposes of this section, unbuildable sensitive areas shall include wetlands, streams and slopes forty percent or steeper and associated buffers; or

b.  In the R-4 through R-8 zones when exceeding base density.

5.  ((Only subject to the residential density incentive provisions of K.C.C. 21A.34.)) Apartment units are permitted outright as follows:

a.  In the R-1 zone at base density or lower when at least fifty percent of the site is constrained by unbuildable sensitive areas.  For purposes of this section, unbuildable sensitive areas shall include wetlands, streams and slopes forty percent or steeper and associated buffers; or

b.  In the R-4 through R-8 zones at base density or lower.

6.  Only as an accessory to a school, college/university or church.

7.a.  Accessory dwelling units:

(1)  Only one accessory dwelling per primary single detached dwelling unit;

(2)  Only in the same building as the primary dwelling unit when the lot is less than ((10,000)) ten thousand square feet in area or when there is more than one primary dwelling on a lot;

(3)  The primary dwelling unit or the accessory dwelling unit shall be owner occupied;

(4)  (a.)  One of the dwelling units shall not exceed a floor area of ((1,000)) one thousand square feet except when one of the dwelling units is wholly contained within a basement or attic, and

(b.)  When the primary and accessory dwelling units are located in the same building, only one entrance may be located on each street side of the building;

(5)  One additional off-street parking space shall be provided;

(6)  The accessory dwelling unit shall be converted to another permitted use or shall be removed if one of the dwelling units ceases to be owner occupied; and

(7)  An applicant seeking to build an accessory dwelling unit shall file a notice approved by the department with the records and elections division which identifies the dwelling unit as accessory.  The notice shall run with the land.  The applicant shall submit proof that the notice was filed before the department shall approve any permit for the construction of the accessory dwelling unit.  The required contents and form of the notice shall be set forth in administrative rules.

(8)  Accessory dwelling units and accessory living quarters are not allowed in the F zone.

b.  One single or twin engine, noncommercial aircraft shall be permitted only on lots which abut, or have a legal access which is not a county right-of-way, to a waterbody or landing field, provided:

(1)  No aircraft sales, service, repair, charter or rental; and

(2)  No storage of aviation fuel except that contained in the tank or tanks of the aircraft.

c.  Buildings for residential accessory uses in the RA and A zone, except for buildings related to agriculture or forestry for which a farm or forestry management plan has been approved by King County department of natural resources, shall not exceed five thousand square feet of gross floor area.

8.  Mobile home parks shall not be permitted in the R-1 zones.

9.  Only as an accessory to the permanent residence of the operator, provided:

a.  Serving meals to paying guests shall be limited to breakfast; and

b.  No more than ((5)) five guests per night.

10.  Only as an accessory to the permanent residence of the operator, provided:

a.  Serving meals to paying guests shall be limited to breakfast; and

b.  The number of persons accommodated per night shall not exceed five, except that a structure which satisfies the standards of the Uniform Building Code as adopted by King County for R-1 occupancies may accommodate up to ten persons per night.

11.  Only when part of a mixed-use development, and subject to the conditions of K.C.C. 21A.08.030B.10.

12.  ((A conditional use permit is not required for townhouse units on lots in a subdivision designed for townhouse units;)) Townhouses are permitted subject to a conditional use permit when exceeding base density.

13.  Required prior to approving more than one dwelling on individual lots, except on lots in subdivisions, short subdivisions, or binding site plans approved for multiple unit lots, and except as provided for accessory dwelling units in K.C.C. 21A.08.030B7.

14.  No new mobile home parks are allowed in a rural zone.

SECTION 9.  Ordinance 10870, Section 331, as amended, and K.C.C. 21A.08.040 are each hereby amended to read as follows:

A.  Recreational/cultural land uses.

KEY

P- Permitted Use

C- Conditional Use

S- Special Use

GENERAL CROSS REFERENCES: 	Land Use Table Instructions, see K.C.C. 21A.08.020 and 21A.02.070; Development Standards, see K.C.C. 21A.12 through 21A.30; General Provisions, see K.C.C. 21A.32 through 21A.38 Application and Review Procedures, see K.C.C. 21A.40 through 21A.44; (*) Definition of this specific land use, see K.C.C. 21A.06.

B.  Development conditions.

1.  The following conditions and limitations shall apply, where appropriate:

a.  No stadiums on sites less than ten acres;

b.  Lighting for structures and fields shall be directed away from residential areas;

c.  Structures or service yards shall maintain a minimum distance of fifty feet from property lines adjoining residential zones, except for structures in on-site recreation areas required in K.C.C. 21A.14.180 and K.C.C. 21A.14.190.  Setback requirements for structures in these on-site required recreation areas shall be maintained pursuant to K.C.C. 21A.12.030;

d.  Facilities in the RA-10, RA-20, F, A or M zones, or in a designated rural ((farm or)) forest ((district)) focus area, shall be limited to trails and trailheads and active recreation facilities, including related accessory uses such as parking and sanitary facilities.  Active recreation facilities shall be limited to those properties within the agricultural production district (APD) that are acquired before designation of the APD, using voter-approved recreation funds, state funds mandated for recreation funds or King County board of recreation funds.  Active recreation uses allowed on parcels as noted in this subsection B.1d may be transferred to other parcels within the same APD.  However, active recreation from lands outside of the APD shall not be relocated to any parcel within an APD.  Where such facilities are permitted within an APD, the following deed restrictions will be applied:

(1)  Active recreation uses shall be designed in a manner that visually screens adjacent agricultural uses from park users and that restricts physical trespass onto adjacent agricultural production district properties;

(2)  Buildings associated with recreational uses shall be limited to restroom facilities, picnic shelters and storage/maintenance facilities for equipment used on-site;

(3)  No use that permanently compacts, removes, sterilizes, pollutes or otherwise materially impairs the future use of the soil for raising agricultural crops shall be allowed;

(4)  Any soil surfaces temporarily disturbed through construction activities shall be restored in a manner consistent with agricultural uses, including restoration of the original soil horizon sequence, as soon as practical following such disruptions;

(5)  Access to recreational uses shall be designed to minimize impact on the surrounding agricultural production district and should be limited to direct access along district boundaries whenever feasible; and

(6)  Although the recreational use of agricultural production district properties may be long term, such use shall be recognized as an interim use of the production district’s prime agricultural soils.  As such, any acquisition funding or policy restrictions for the recreational use of the property shall be viewed as subordinate to the county’s prior commitment to the preservation of prime agricultural soils and the viability of local agricultural production.  Whenever the county declares through action of the King County council a critical shortage of agricultural soils to accommodate an active soil-dependent agricultural proposal, the county shall initiate a process to relocate any recreational uses off the subject property and to make the property available for re-establishment of agricultural activities; and

e.  Overnight camping is allowed only in an approved campground.

2.  Recreational vehicle parks are subject to the following conditions and limitations:

a.  The maximum length of stay of any vehicle shall not exceed one hundred eighty days during a three hundred sixty-five day period;

b.  The minimum distance between recreational vehicle pads shall be no less than ten feet; and

c.  Sewage shall be disposed in a system approved by the Seattle-King County health department.

3.  Limited to day moorage.  The marina shall not create a need for off-site public services beyond those already available before the date of application.

4.  Not permitted in the RA-10 or RA-20 zones.((, or in the RA-10 zone when located in a designated rural farm district, l))Limited to recreation facilities subject to the following conditions and limitations:

a.  The bulk and scale shall be compatible with residential or rural character of the area;

b.  For sports clubs, the gross floor area shall not exceed ten thousand square feet unless the building is on the same site or adjacent to a site where a public facility is located or unless the building is a nonprofit facility located in the urban area; and

c.  Use is limited to residents of a specified residential development or to sports clubs providing supervised instructional or athletic programs.

5.  Limited to day moorage.

6.  a.  Adult entertainment businesses shall be prohibited within three hundred thirty feet of any property zoned RA, UR or R or containing schools licensed daycare centers, public parks or trails, community centers, public libraries or churches.  In addition, adult entertainment businesses shall not be located closer than three thousand feet to any other adult entertainment business.  These distances shall be measured from the property line of the parcel or parcels proposed to contain the adult entertainment business to the property line of the parcels zoned RA, UR or R or that contain the uses identified in this subsection B.6a.

b.  Adult entertainment businesses shall not be permitted within an area likely to be annexed to a city subject to an executed interlocal agreement between King County and a city declaring that the city will provide opportunities for the location of adult businesses to serve the area.  The areas include those identified in the maps attached to Ordinance 13546.

7.  Clubhouses, maintenance buildings and equipment storage areas, and driving range tees shall be at least fifty feet from residential property lines. Lighting for practice greens and driving range ball impact areas shall be directed away from adjoining residential zones.  Applications shall comply with adopted best management practices for golf course development.  ((The department shall determine by administrative rule best management practices for golf course development.  These rules shall be delivered to the council for review by July 31, 1998.))  Within the RA zone, such facilities shall be permitted only in the RA-5((,)) and RA-2.5 ((and RA-10)) zones.  Not permitted in ((a)) designated ((rural farm or forest district,)) regionally significant resource areas or locally significant resource areas.  ((In the RA-10 zone, a))Ancillary facilities associated with a golf course are limited to practice putting greens, maintenance buildings and other structures housing administrative offices or activities which provide convenience services to players.  These convenience services are limited to a pro shop, food services and dressing facilities and shall occupy a total of no more than ten thousand square feet. Furthermore, the residential density that is otherwise permitted by the zone shall not be used on other portions of the site through clustering or on other sites through the transfer of density provision.  This residential density clustering or transfer limitation shall be reflected in a deed restriction that is recorded at the time applicable permits for the development of the golf course are issued.  ((In the RA-10 zone, the county shall limit this use to no more than six development permit applications of which only three may be over one hundred acres in size.  The provisions permitting this use expire on April 1, 2003, unless re-enacted by council action.))

8.  Limited to a golf driving range as an accessory to golf courses.

9.  a.  New structures and outdoor ranges shall maintain a minimum distance of fifty feet from property lines adjoining residential zones, but existing facilities shall be exempt;

b.  Ranges shall be designed to prevent stray or ricocheting projectiles, pellets or arrows from leaving the property;

c.  Site plans shall include:  safety features of the range; provisions for reducing sound produced on the firing line; elevations of the range showing target area, backdrops or butts; and approximate locations of buildings on adjoining properties; and

d.  Subject to the licensing provisions of K.C.C. Title 6.

10.  a.  Only in an enclosed building, and subject to the licensing provisions of K.C.C. Title 6;

b.  Indoor ranges shall be designed and operated so as to provide a healthful environment for users and operators by:

(1) installing ventilation systems which provide sufficient clean air in the user's breathing zone, and

(2) adopting appropriate procedures and policies which monitor and control exposure time to airborne lead for individual users.

11.  Only as accessory to a park or in a building listed on the National Register as an historic site or designated as a King County landmark subject to K.C.C. chapter 21A.32.

12.  Only as accessory to a nonresidential use established through a discretionary permit process, and provided further that the scale is limited to ensure compatibility with surrounding neighborhoods, and provided further that this condition applies to the UR zone only if the property is located within a designated unincorporated ((r))Rural ((t))Town.

13.  Subject to the following:

a.  The park shall abut an existing park on one or more sides, intervening roads notwithstanding;

b.  No bleachers or stadiums are permitted if the site is less than ten acres, and no public amusement devices for hire are permitted;

c.  Any lights provided to illuminate any building or recreational area shall be so arranged as to reflect the light away from any premises upon which a dwelling unit is located; and

d.  All buildings or structures or service yards on the site shall maintain a distance not less than fifty feet from any property line and from any public street.

14.  Excluding amusement and recreational uses classified elsewhere in this chapter.

15.  Limited to golf driving ranges and subject to K.C.C. 21A.08.040B.7.

16.  Subject to the following conditions:

a.  The length of stay per party in campgrounds shall not exceed one hundred eighty days during a three hundred sixty-five day period; and

b.  Only for campgrounds that are part of a proposed or existing county park, which are subject to review and public hearings through the department of parks and recreation's master plan process under K.C.C. 2.16.050.

17.  Only for stand-alone sports clubs that are not part of a park.

SECTION 10.  Ordinance 10870, Section 332, as amended, and K.C.C. 21A.08.050 are each hereby amended to read as follows:

A.  General ((S))services land uses.

KEY

P- Permitted Use

C- Conditional Use

S- Special Use

GENERAL CROSS REFERENCES:	Land Use Table Instructions, see K.C.C. 21A.08.020 and 21A.02.070;  Development Standards, see K.C.C. 21A.12 through 21A.30;  General Provisions, see K.C.C. 21A.32 through 21A.38; Application and Review Procedures, see K.C.C. 21A.40 through 21A.44; (*) Definition of this specific Land Use, see K.C.C. 21A.06

B.  Development conditions.

1.  Except SIC Industry No. 7534 - Tire Retreading, see manufacturing permitted use table.

2.  Except SIC Industry Group Nos.:

a.  835-Day Care Services, and

b.  836-Residential Care, which is otherwise provided for on the residential permitted land use table.

3.  Limited to SIC Industry Group and Industry Nos.:

a.  723-Beauty Shops;

b.  724-Barber Shops;

c.  725-Shoe Repair Shops and Shoeshine Parlors;

d.  7212-Garment Pressing and Agents for Laundries and Drycleaners;

e.  217-Carpet and Upholstery Cleaning.

4.  Only as an accessory to a cemetery, and prohibited from the UR zone only if the property is located within a designated unincorporated Rural Town.

5.  Structures shall maintain a minimum distance of one hundred feet from property lines adjoining residential zones.

6.  Only as an accessory to residential use, provided:

a.  Outdoor play areas shall be completely enclosed by a solid wall or fence, with no openings except for gates, and have a minimum height of six feet; and

b.  Outdoor play equipment shall maintain a minimum distance of twenty feet from property lines adjoining residential zones.

7.  Permitted as an accessory use, see commercial/industrial accessory, K.C.C. 21A.08.060A.

8.  Only as a re-use of a public school facility subject to the provisions of K.C.C. chapter 21A.32, or an accessory use to a school, church, park, sport club or public housing administered by a public agency, provided:

a.  Outdoor play areas shall be completely enclosed by a solid wall or fence, with no openings except for gates and have a minimum height of six feet;

b.  Outdoor play equipment shall maintain a minimum distance of twenty feet from property lines adjoining residential zones;

c.  Direct access to a developed arterial street shall be required in any residential zone; and

d.  Hours of operation may be restricted to assure compatibility with surrounding development.

9.  As a home occupation only, provided that the square footage limitations in K.C.C. chapter 21A.30 for home occupations apply only to the office space for the clinic, and provided further that:

a.  Boarding or overnight stay of animals is allowed only on sites of five acres or more;

b.  No burning of refuse or dead animals is allowed;

c.  The portion of the building or structure in which animals are kept or treated shall be soundproofed.  All run areas, excluding confinement areas for livestock, shall be surrounded by an eight-foot solid wall and surfaced with concrete or other impervious material; and

d.  The provisions of K.C.C. chapter 21A.30 relative to animal keeping are met.

10.  a.  No burning of refuse or dead animals is allowed;

b.  The portion of the building or structure in which animals are kept or treated shall be soundproofed.  All run areas, excluding confinement areas for livestock, shall be surrounded by an eight-foot solid wall and surfaced with concrete or other impervious material; and

c.  The provisions of K.C.C. chapter 21A.30 relative to animal keeping are met.

11.  The repair work or service shall only be performed in an enclosed building, and no outdoor storage of materials.  SIC Industry No. 7532 - Top, Body, and Upholstery Repair Shops and Paint Shops is not allowed.

12.  Only as a re-use of a public school facility subject to the provisions of K.C.C. chapter 21A.32.

13.  Only as a re-use of a surplus nonresidential facility subject to K.C.C. chapter 21A.32.

14.  Covered riding arenas are subject to the provisions of K.C.C. 21A.30.030 and shall not exceed twenty thousand square feet, provided that stabling areas, whether attached or detached, shall not be counted in this calculation.

15.  Limited to ((projects)) schools ((which)) that do not require or result in an expansion of sewer service outside the urban growth area((, unless a finding is made that no cost-effective alternative technologies are feasible, in which case a tightline sewer sized only to meet the needs of the school and serving only the school may be used)), except for public schools, as defined by RCW 28A.150.010.  Under no circumstances shall other uses or structures in the rural area be connected to and served by the public school's tightline sewer.  New public high schools shall be permitted subject to the review process set forth in K.C.C. 21A.42.140.

16.  a.  For middle/junior high schools and secondary or high schools, only as a re-use of a public school facility subject to the provisions of K.C.C. chapter 21A.32.  An expansion of such school facility shall be subject to approval of a conditional use permit and the expansion shall not require or result in an extension of sewer service outside the urban growth area ((, unless a finding is made that no cost-effective alternative technologies are feasible)), except for public schools, as defined by RCW 28A.150.010, in which case a tightline sewer sized only to meet the needs of the public school may be used.

b.  Renovation, expansion, modernization or reconstruction of a school, or the addition of relocatable facilities, is permitted but shall not require or result in an expansion of sewer service outside the urban growth area ((, unless a finding is made that no cost-effective alternative technologies are feasible)), except for public schools, as defined by RCW 28A.150.010, in which case a tightline sewer sized only to meet the needs of the public school may be used.

17.  All instruction must be within an enclosed structure.

18.  Limited to resource management education programs.

19.  Only as an accessory to residential use, provided:

a.  Students are limited to twelve per one-hour session,

b.  All instruction must be within an enclosed structure, and

c.  Structures used for the school shall maintain a distance of twenty-five feet from property lines adjoining residential zones.

20.  Subject to the following:

a.  Structures used for the school and accessory uses shall maintain a minimum distance of twenty-five feet from property lines adjoining residential zones;

b.  On lots over two and one half acres:

(1) Retail sales of items related to the instructional courses is permitted, provided total floor area for retail sales is limited to two thousand square feet;

(2) Sales of food prepared in the instructional courses is permitted with Seattle-King County health department approval, provided total floor area for food sales is limited to one thousand square feet and is located in the same structure as the school; and

(3) Other incidental student-supporting uses are allowed, provided such uses are found to be both compatible with and incidental to the principal use; and

c.  On sites over ten acres, located in a designated Rural Town and zoned UR, R-1, and/or R-4:

(1) Retail sales of items related to the instructional courses is permitted, provided total floor area for retail sales is limited to two thousand square feet;

(2) Sales of food prepared in the instructional courses is permitted with Seattle-King County health department approval, provided total floor area for food sales is limited to one thousand seven hundred fifty square feet and is located in the same structure as the school;

(3) Other incidental student-supporting uses are allowed, provided such uses are found to be functionally related, subordinate, compatible with and incidental to the principal use;

(4) The use is integrated with allowable agricultural uses on the site;

(5) Advertised special events shall comply with the temporary use requirements of this chapter; and

(6) Existing structures that are damaged or destroyed by fire or natural event, if damaged by more than fifty percent of their prior value, may reconstruct and expand an additional sixty-five percent of the original floor area but need not be approved as a conditional use if their use otherwise complies with the standards set forth in development condition B20c of this section and the requirements of this title.

21.  Limited to source-separated yard or organic waste processing facilities.

22.  Limited to drop box facilities accessory to a public or community use such as a school, fire station or community center.

23.  With the exception of drop box facilities for the collection and temporary storage of recyclable materials, all processing and storage of material shall be within enclosed buildings.  Yard waste processing is not permitted.

24.  Only when adjacent to an existing or proposed school.

25.  Limited to columbariums accessory to a church provided that required landscaping and parking are not reduced.

26.  Not permitted in R-1 and limited to a maximum of five thousand square feet per establishment and subject to the additional requirements in K.C.C. 21A.12.230.

27.  a.  New high schools shall be permitted in urban residential and urban reserve zones subject to the review process set forth in K.C.C. 21A.42.140 and

b.  Renovation, expansion, modernization, or reconstruction of a school, or the addition of relocatable facilities, is permitted.

28.  Limited to projects which do not require or result in an expansion of sewer service outside the urban growth area.  In addition, such use shall not be permitted in the RA-20 zone.

29.  Only as a reuse of a surplus non-residential facility subject to K.C.C. chapter 21A.32 or as a joint use of an existing public school facility.

30.  All studio use must be within an enclosed structure.

31.  Adult use facilities shall be prohibited within six hundred sixty feet of any residential zones, any other adult use facility, or school licensed daycare centers, parks, community centers, public libraries or churches which conduct religious or educational classes for minors.

SECTION 11.  Ordinance 10870, Section 334, as amended, and K.C.C. 21A.08.070 are each hereby amended to read as follows:

A.  Retail land uses.

General Cross References:	Land Use Table Instructions, see K.C.C. 21A.08.020 and 21A.02.070; Development Standards, see K.C.C. 21A.12 through 21A.30; General Provisions, see K.C.C. 21A.32 through 21A.38; Application and Review procedures, see K.C.C. 21A.40 through 21A.44; (*) Definition of this specific land use, see K.C.C. 21A.06.

KEY

P- Permitted Use

C- Conditional Use

S- Special Use

B.  Development conditions.

1.  Only feed stores and garden supply stores.

2.  Only hardware and garden materials stores shall be permitted.

3.  a.  ((Except for hay sales, l))Limited to products produced on-site; and

b.  Covered sales areas shall not exceed a total area of five hundred square feet.

4.  No permanent structures or signs.

5.  Limited to SIC Industry No. 5331, Variety Stores, and further limited to a maximum of two thousand square feet of gross floor area.

6.  Limited to a maximum of two thousand square feet of gross floor area.

7.  a.  The floor area devoted to retail sales shall not exceed two thousand five hundred square feet;

b.  ((No more than forty)) Sixty percent or more of the average annual gross sales of agricultural products sold through the store over a five-year period shall be derived from products ((not)) grown or produced in King County.  At the time of the initial application, the applicant shall submit a reasonable projection of the source of product sales;

c.  Sales shall be limited to agricultural produce and plants;

d.  Storage areas for produce may be included in a farm store structure or in any accessory building;

e.  Hours of operation shall be limited to 7:00 a.m. to 9:00 p.m. during the months of May through September and 7:00 a.m. to 7:00 p.m. during the months of October through April; outside lighting is permitted if no off-site glare is allowed((; and

f.  Noncontiguous lands within King County may be assembled by an individual farmer or group of farmers for the purposes of establishing a source of local products to be sold in a farm store on one of the properties)).

8.  Excluding retail sale of trucks exceeding one-ton capacity.

9.  Only the sale of new or reconditioned automobile supplies is permitted.

10.  Excluding SIC Industry No. 5813-Drinking Places.

11.  No outside storage of fuel trucks and equipment.

12.  Excluding vehicle and livestock auctions.

13.  Only as accessory to a winery or brewery.

14.  Not in R-1 and limited to SIC Industry No. 5331 - Variety Stores, limited to a maximum of five thousand square feet of gross floor area, and subject to K.C.C. 21A.12.330.

15.  Not permitted in R-1 and limited to a maximum of five thousand square feet of gross floor area and subject to K.C.C. 21A.12.230.

16.  Not permitted in R-1 and excluding SIC Industry No. 5813-Drinking Places, and limited to a maximum of five thousand square feet of gross floor area and subject to K.C.C. 21A.12.230.

17.  Retail sale of livestock is permitted only as accessory to raising livestock.

18.  Limited to the R-1 zone.

19.  Limited to the sale of livestock feed, hay and livestock veterinary supplies with a covered sales area of not more than five hundred square feet.

20.a.  Covered sales areas shall not exceed a total area of one thousand two hundred fifty square feet;

b.  Sixty percent or more of the average annual gross sales of agricultural products sold through the store over a five-year period shall be derived from products grown or produced in King County.  At the time of the initial application, the applicant shall submit a projection of the source of product sales;

c.  Sales shall be limited to agricultural produce and plants;

d.  Storage areas for produce may be included in a farm store structure or in any accessory building; and

e.  Hours of operation shall be limited to 7:00 a.m. to 9:00 p.m. during the months of May through September and 7:00 a.m. to 7:00 p.m. during the months of October through April.  Outside lighting is permitted if no off-site glare is allowed.

21.  Limited to hay sales.

SECTION 12.  Ordinance 10870, Section 335, as amended, and K.C.C. 21A.08.080 are each hereby amended to read as follows:

A.  Manufacturing land uses.

| K E Y |  |  |  |  |  | RESOURCE |  |  |  | RESIDENTIAL |  |  |  |  |  |  |  |  |  |  |  | COMMERCIAL/ INDUSTRIAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-Permitted Use C-Conditional Use |  |  |  |  |  | A | F | M |  | R | U |  | R |  |  | U |  | R |  |  |  | N |  |  | B |  | C |  | B | R |  | B | O |  | I |  |
| S-Special Use |  |  |  |  |  | G | O | I |  | U | R |  | E |  |  | R |  | E |  |  |  | E |  |  | U |  | O |  | U | E |  | U | F |  | N |  |
|  |  |  |  |  | Z | R | R | N |  | R | B |  | S |  |  | B |  | S |  |  |  | I |  |  | S |  | M |  | S | G |  | S | F |  | D |  |
|  |  |  |  |  | O | I | E | E |  | A | A |  | E |  |  | A |  | I |  |  |  | G |  |  | I |  | M |  | I | I |  | I | I |  | U |  |
|  |  |  |  |  | N | C | S | R |  | L | N |  | R |  |  | N |  | D |  |  |  | H |  |  | N |  | U |  | N | O |  | N | C |  | S |  |
|  |  |  |  |  | E | U | T | A |  |  |  |  | V |  |  |  |  | E |  |  |  | B |  |  | E |  | N |  | E | N |  | E | E |  | T |  |
|  |  |  |  |  |  | L |  | L |  |  |  |  | E |  |  |  |  | N |  |  |  | O |  |  | S |  | I |  | S | A |  | S |  |  | R |  |
|  |  |  |  |  |  | T U |  |  |  |  |  |  |  |  |  |  | T I |  |  |  |  | R H |  | S |  |  | T Y | S |  | L |  | S |  |  | I A |  |
|  |  |  |  |  |  | R |  |  |  |  |  |  |  |  |  |  | A |  |  |  |  | O |  |  |  |  |  |  |  |  |  |  |  |  | L |  |
|  |  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  | L |  |  |  |  | O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIC# | SPECIFIC LAND USE |  |  |  |  | A | F | M |  | RA | UR |  |  | R1-8 |  |  |  |  | R12-48 |  |  |  | NB |  |  | CB |  |  |  |  | RB |  |  | O |  | I (11) |
| 20 | Food and Kindred Products |  |  |  |  | P1 | P1 |  |  | P1 | P1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P2 C |
| 2082/ 2084 | Winery/Brewery |  |  |  |  | P1 |  |  |  | P3 | P3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| 22 | Textile Mill Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 23 | Apparel and other Textile Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| 24 | Wood Products, except furniture |  |  |  |  | P4 | P4 C5 |  |  | P4 | P4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C6 |  |  |  |  | P |
| 25 | Furniture and Fixtures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| 26 | Paper and Allied Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 27 | Printing and Publishing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P7 |  |  | P7 |  |  |  |  | P7C |  |  | P7C |  | P |
| 28 | Chemicals and Allied Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 2911 | Petroleum Refining and related industries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 30 | Rubber and Misc. Plastics Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 31 | Leather and Leather Goods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| 32 | Stone, Clay, Glass and Concrete Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P8 |  |  |  |  | P9 |  |  |  |  | P |
| 33 | Primary Metal Industries |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 34 | Fabricated Metal Products |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P |
| 35 | Industrial and Commercial Machinery |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P |
| 351-55 | Heavy Machinery and Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 357 | Computer and Office Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  | C |  | P |
| 36 | Electronic and other Electric Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| 374 | Railroad Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 376 | Guided Missile and Space Vehicle Parts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 379 | Miscellaneous Transportation Vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| 38 | Measuring and Controlling Instruments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  | C |  | P |
| 39 | Miscellaneous Light Manufacturing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| * | Motor Vehicle and Bicycle Manufacturing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |
| * | Aircraft, Ship and Boat Building |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P10 C |
| 7534 | Tire Retreading |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | C |  |  |  |  | P |
| 781-82 | Movie Production/Distribution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P |  |  |  |  | P |
| GENERAL CROSS REFERENCES: Land Use Table Instructions, see K.C.C. 21A.08.020 and 21A.02.070; Development Standards, see K.C.C. 21A.12 through 21A.30; General Provisions, see K.C.C. 21A.32 through 21A.38; Application and Review Procedures, see K.C.C. 21A.40 through 21A.44; (*)Definition of this specific land use, see K.C.C. 21A.06. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B.  Development conditions.

1.  Limited to agricultural products produced on-site, provided structures and areas used for processing shall maintain a minimum distance of ((75)) seventy-five feet from property lines adjoining residential zones.

2.  Except slaughterhouses.

3.  Only as a home industry, subject to K.C.C. chapter 21A.30.

4.  Limited to rough milling and planing of products grown on-site with portable equipment.

5.  Limited to SIC Industry Group No. 242-Sawmills.

6.  Limited to uses found in SIC Industry No. 2434-Wood Kitchen Cabinets and No. 2431-Millwork (excluding planing mills).

7.  Limited to photocopying and printing services offered to the general public.

8.  Only within enclosed buildings, and as an accessory use to retail sales.

9.  Only within enclosed buildings.

10.  Limited to boat building of craft not exceeding ((48)) forty-eight feet in length.

11.  For I-zoned sites located outside the urban growth area designated by the King County Comprehensive Plan, uses shown as a conditional use in the table of K.C.C. 21A.08.080A shall be prohibited, and all other uses shall be subject to the provisions for rural industrial uses as set forth in K.C.C. chapter 21A.12.

SECTION 13.  Ordinance 10870, Section 336, as amended, and K.C.C. 21A.08.090 are each hereby amended to read as follows:

A.  Resource land uses.

GENERAL CROSS REFERENCES:		Land Use Table Instructions, see K.C.C. 21A.08.020 and

Development Standards, see K.C.C. 21A.12 through 21A.30;

General Provisions, see K.C.C. 21A.32 through 21A.38;

Application and Review Procedures, see K.C.C. 21A.40

(*) Definition of this specific land use, see K.C.C> 21A.06.

KEY

P- Permitted Use

C- Conditional Use

S- Special Use

B.  Development conditions.

1.  May be further subject to the provisions of K.C.C. Title 25, Shoreline Management Program.

2.  Only forest research conducted within an enclosed building.

3.  ((Housing))Accessory dwelling units for agricultural workers and their families employed on the premises only as an accessory to a permanent residence, provided:

a.  Sites are limited to those with ten acres or more;

b.  Accessory ((housing is)) dwelling units are limited to one unit on less than ((20)) twenty acres, or two on ((20)) twenty acres or more; ((and))

c.  Accessory ((housing)) dwelling units shall not be rented to the public at large; and

d.  Accessory dwelling units must be constructed in conformance with the State Building Code.

4.  Excluding housing for agricultural workers.

5.  Limited to maintenance and/or storage facilities in conjunction with mineral extraction or processing operation.

6.  Large livestock allowed only in the R-1 zone.

7.  Only in conjunction with a mineral extraction site plan approved pursuant to K.C.C. chapter 21A.22.

8.  Only as accessory to a primary mineral extraction use, or as a continuation of a mineral processing use established prior to the effective date of or consistent with this code.

9.  Limited to mining activities that are located greater than one-quarter mile from an established residence and that do not use local access streets which abut lots developed for residential use.

10.  Agriculture training facilities are allowed only as an accessory to existing agricultural uses and are subject to the following conditions:

a.  The impervious surface associated with the agriculture training facilities shall comprise not more than ((10)) ten percent of the allowable impervious surface permitted under K.C.C. 21A.12.040;

b.  New or the expansion of existing structures, or other site improvements, shall not be located on class 1, 2, or 3 soils;

c.  The director may require reuse of surplus structures to the maximum extent practical;

d.  The director may require the clustering of new structures with existing structures;

e.  New structures or other site improvements shall be set back a minimum distance of ((75)) seventy-five feet from property lines adjoining residential zones;

f.  Bulk and design of structures shall be compatible with the architectural style of the surrounding agricultural community;

g.  New sewers shall not be extended to the site;

h.  Traffic generated shall not impede the safe and efficient movement of agricultural vehicles, nor shall it require capacity improvements to rural roads;

i.  Agriculture training facilities may be used to provide educational services to the surrounding rural/agricultural community or for community events. Property owners may be required to obtain a temporary use permit for community events in accordance with K.C.C. chapter 21A.32;

j.  Use of lodging and food service facilities shall be limited only to activities conducted in conjunction with training and education programs or community events held on site;

k.  Incidental uses, such as office and storage, shall be limited to those which directly support education and training activities or farm operations; and

l.  The King County agriculture commission shall be notified of and have an opportunity to comment upon all proposed agriculture training facilities during the permit process in accordance with K.C.C. chapter 21A.40.

SECTION 14.  Ordinance 10870, Section 337, as amended, and K.C.C. 21A.08.100 are each hereby amended to read as follows:

A.  Regional land uses.

GENERAL CROSS REFERENCES:		Land Use Table Instructions, see K.C.C. 21A.08.020 and

Development Standards, see K.C.C. 21A.12 through 21A.30;

General Provisions, see K.C.C. 21A.32 through 21A.38;

Application and Review Procedures, see K.C.C. 21A.40

(*) Definition of this specific land use, see K.C.C> 21A.06.

KEY

P- Permitted Use

C- Conditional Use

S- Special Use

B.  Development Conditions.

1.  Except technical institutions.  See vocational schools on general services land use table, K.C.C. 21A.08.050.

2.  Except arboretum.  See K.C.C. 21A.08.040, recreation/cultural land use table.

3.  Except weapons armories and outdoor shooting ranges.

4.  Except outdoor shooting range.

5.  Only ((in conjunction with)) on the same lot or on an abutting lot of an existing or proposed school.

6.  a.  Limited to no more than three satellite dish antennae.

b.  Limited to one satellite dish antenna.

c.  Limited to tower consolidations.

7.  Limited to landing field for aircraft involved in forestry or agricultural practices or for emergency landing sites.

8.  Except racing of motorized vehicles.

9.  Limited to wildlife exhibit.

10.  Only as a re-use of a public school facility subject to the provisions of K.C.C. chapter 21A.32.

11.  Only as a re-use of a surplus nonresidential facility subject to the provisions of K.C.C. chapter 21A.32.

12.  Limited to cogeneration facilities for on-site use only.

13.  Excluding impoundment of water using a dam.

14.  Limited to facilities that comply with the following provisions:

a.  Any new diversion structure shall not:

(1)  exceed a height of eight feet as measured from the streambed, or

(2)  impound more than three surface acres of water at the normal maximum surface level;

b.  There shall be no active storage;

c.  The maximum water surface area at any existing dam or diversion shall not be increased;

d.  An exceedance flow of no greater than fifty percent in mainstream reach shall be maintained;

e.  Any transmission line shall be limited to a:

(1)  right-of-way of five miles or less; and

(2)  capacity of two hundred thirty KV or less;

f.  Any new, permanent access road shall be limited to five miles or less; and

g.  The facility shall only be located above any portion of the stream used by anadromous fish.

15.  For I-zoned sites located outside the urban growth area designated by the King County Comprehensive Plan, uses shown as a conditional or special use in K.C.C. 21A.08.100A, except for waste water treatment facilities, shall be prohibited. All other uses, including waste water treatment facilities, shall be subject to the provisions for rural industrial uses as set forth in K.C.C. chapter 21A.12.

16.  The operator of such facility shall provide verification to the department of natural resources or its successor organization that the facility meets or exceeds the standards of the Animal and Plant Health Inspection Service of the United States Department of Agriculture and the accreditation guidelines of the American Zoo and Aquarium Association.

17.  The following provisions of the table apply only to major communication facilities ((M))minor communication facilities shall be reviewed pursuant to the processes and standard outlined in K.C.C. chapter 21A.26.

18.  Only for facilities related to resource-based research.

19.  Limited to work release facilities associated with natural resource-based activities.

SECTION 15.  Ordinance 10870, Section 340, as amended, and K.C.C. 21A.12.030 are each hereby amended to read as follows:

A.  Densities and dimensions - residential zones

B.  Development conditions.

1.  This maximum density may be achieved only through the application of residential density incentives pursuant to K.C.C. chapter 21A.34 or transfers of density credits pursuant to K.C.C. chapters 21A.36 or 21A.55, or any combination of density incentive or density transfer.  Maximum density may only be exceeded pursuant to K.C.C. 21A.34.040F.1.((f.))g.

2.  Also see K.C.C. 21A.12.060.

3.  These standards may be modified under the provisions for zero-lot-line and townhouse developments.

4.  Height limits may be increased when portions of the structure which exceed the base height limit provide one additional foot of street and interior setback for each foot above the base height limit, provided the maximum height may not exceed seventy-five feet.  Netting or fencing and support structures for the netting or fencing used to contain golf balls in the operation of golf courses or golf driving ranges are exempt from the additional interior setback requirements provided that the maximum height shall not exceed seventy-five feet.

5.  Applies to each individual lot. Impervious surface area standards for:

a.  regional uses shall be established at the time of permit review;

b.  nonresidential uses in residential zones shall comply with  K.C.C. 21A.12.120 and 21A.12.220;

c.  individual lots in the R-4 through R-6 zones which are less than nine thousand seventy-six square feet in area shall be subject to the applicable provisions of the nearest comparable R-6 or R-8 zone;

d.  lot may be increased beyond the total amount permitted in this chapter subject to approval of a conditional use permit.

6.  Mobile home parks shall be allowed a base density of six dwelling units per acre.

7.  The standards of the R-4 zone shall apply if a lot is less than fifteen thousand square feet in area.

8.  At least twenty linear feet of driveway shall be provided between any garage, carport or other fenced parking area and the street property line.  The linear distance shall be measured along the center line of the driveway from the access point to such garage, carport or fenced area to the street property line.

9.  a.  Residences shall have a setback of at least one hundred feet from any property line adjoining A, M or F zones or existing extractive operations.  However, residences on lots less than one hundred fifty feet in width adjoining A, M or F zone or existing extractive operations shall have a setback equal to fifty percent of the lot width.

b.  Except for residences along a property line adjoining A, M or F zones or existing extractive operations, for lots between one acre and two and one-half acres in size, the requirements of the R-1 zone shall apply((. For)) and for lots under one acre, the requirements of the R-4 zone shall apply.

10.  a.  For developments consisting of three or more single-detached dwellings located on a single parcel, the setback shall be ten feet along any property line abutting R-1 through R-8, RA and UR zones, except for structures in on-site play areas required in K.C.C. 21A.14.190, which shall have a setback of five feet.

b.  for townhouse and apartment development, the setback shall be twenty feet along any property line abutting R-1 through R-8, RA and UR zones, except for structures in on-site play areas required in K.C.C. 21A.14.190, which shall have a setback of five feet, unless the townhouse or apartment development is adjacent to property upon which an existing townhouse or apartment development is located.

11.  Lots smaller than one-half acre in area shall comply with standards of the nearest comparable R-4 through R-8 zone.  For lots that are one-half acre in area or larger, the maximum impervious surface area allowed shall be at least ten thousand square feet.  On any lot over one acre in area, an additional five percent of the lot area may be used for buildings related to agricultural or forestry practices. For lots smaller than two acres but larger than one-half acre, an additional ten percent of the lot area may be used for structures which are determined to be medically necessary, provided the applicant submits with the permit application a notarized affidavit, conforming with the requirements of K.C.C. 21A.32.170A.2.

12.  For purposes of calculating minimum density, the applicant may request that the minimum density factor be modified based upon the weighted average slope of the net buildable area(s) of the site pursuant to K.C.C. 21A.12.087.

13.  Reserved.

14.  The base height to be used only for projects as follows:

a.  in R-6 and R-8 zones, a building with a footprint built on slopes exceeding a fifteen percent finished grade; and

b.  in R-18, R-24 and R-48 zones using residential density incentives and transfer of density credits pursuant to this title.

15.  Density applies only to dwelling units and not to sleeping units.  For the purpose of density transfer only, the base density for property located within a rural forest focus area shall be calculated at one dwelling unit per five acres on a sending site that qualifies under the transfer of development credit pilot program in K.C.C. chapter 21A.55.

16.  Vehicle access points from garages, carports or fenced parking areas shall be set back from the property line on which a joint use driveway is located to provide a straight line length of at least twenty-six feet as measured from the center line of the garage, carport or fenced parking area, from the access point to the opposite side of the joint use driveway.

17.  a.  All subdivisions and short subdivisions in the R-1 zone shall be required to be clustered when the property is located within or contains:

(1)  a floodplain,

(2)  a critical aquifer recharge area,

(3)  a Regionally or Locally Significant ((r))Resource Area,

(4)  existing or planned public parks or trails, or connections to such facilities,

(5)  a Class I or II stream or wetland((;)), or

(6)  a "greenbelt/urban separator" or "wildlife corridor" –area designated by the comprehensive plan or a community plan.

b.  The development shall be clustered away from sensitive areas or the axis of designated corridors such as urban separators or the wildlife habitat network to the extent possible and the open space shall be placed in a separate tract that includes at least fifty percent of the site.  Open space tracts shall be permanent and shall be dedicated to a homeowner’s association or other suitable organization, as determined by the director, and meet the provisions of K.C.C. 21A.14.040.  Onsite sensitive area and buffers, wildlife habitat networks, required habitat and buffers for protected species and designated urban separators should be placed within the open space tract to the extent possible.  Passive recreation (with no development of recreational facilities) and natural-surface pedestrian and equestrian trails are acceptable uses within the open space tract.

18.  See K.C.C. 21A.12.085.

19.  All subdivisions and short subdivisions in R-1 and RA zones within the North Fork and Upper Issaquah Creek subbasins of the Issaquah Creek Basin (the North Fork and Upper Issaquah Creek subbasins are identified in the Issaquah Creek Basin and Nonpoint Action Plan) and the portion of the Grand Ridge subarea of the East Sammamish Community Planning Area which drains to Patterson Creek shall have a maximum impervious surface area of eight percent of the gross acreage of the plat.  Distribution of the allowable impervious area among the platted lots shall be recorded on the face of the plat.  Impervious surface of roads need not be counted towards the allowable impervious area.  In cases where both lot- and plat-specific impervious limits apply, the more restrictive shall be required.

20.  This density may only be achieved on RA 2.5 and RA 5 zoned parcels ((designated as rural)) receiving density from rural forest focus areas through the ((T))transfer of ((D))density ((C))credit ((P))pilot ((P))program outlined in K.C.C. chapter 21A.55.

21.  Base density may be exceeded, if the property is located in a designated rural city urban growth area and each proposed lot contains an occupied legal residence which predates 1959.

SECTION 16.  Ordinance 10870, Section 341, as amended, and K.C.C. 21A.12.040 are each hereby amended to read as follows:

A.  Densities and dimensions - resource and commercial/industrial zones.

B.  Development Conditions.

1.  Reserved.

2.  These densities are allowed only through the application of mixed use development standards and for stand-alone townhouse development in the NB zone on property designated commercial outside of center in the urban area.

3.  These densities may only be achieved through the application of residential density incentives or transfer of density credits in mixed use developments and for stand-alone townhouse development in the NB zone on property designated commercial outside of center in the urban area. See K.C.C. chapters 21A.34 and 21A.36.

4.  a.  In the F zone, scaling stations may be located thirty-five feet from property lines.  Residences shall have a setback of at least thirty feet from all property lines.

b.  For lots between one acre and ((2.5)) two and one half acres in size, the setback requirements of the R-1 zone shall apply.  For lots under one acre, the setback requirements of the R-4 zone shall apply.

c.  For developments consisting of three or more single-detached dwellings located on a single parcel, the setback shall be ten feet along any property line abutting R-1 through R-8, RA and UR zones.

5.  Gas station pump islands shall be placed no closer than twenty-five feet to street front lines.

6.  This base height allowed only for mixed use developments and for stand-alone townhouse development in the NB zone on property designated commercial outside of center in the urban area.

7.  Required on property lines adjoining residential zones.

8.  Required on property lines adjoining residential zones for industrial uses established by conditional use permits.

9.  The floor/lot ratio for mixed use developments shall conform to K.C.C. chapter 21A.14.

10.  Height limits may be increased when portions of the structure building which exceed the base height limit provide one additional foot of street and interior setback for each foot above the base height limit, provided the maximum height may exceed seventy-five feet only in mixed use developments.  Netting or fencing and support structures for the netting or fencing used to contain golf balls in the operation of golf courses or golf driving ranges are exempt from the additional interior setback requirement provided that the maximum height shall not exceed seventy-five feet.

11.  Applicable only to lots containing less than one acre of lot area. Development on lots containing less than ((15,000)) fifteen thousand square feet of lot area shall be governed by impervious surface standards of the nearest comparable R-4 through R-8 zone.

12.  See K.C.C. 21A.22.060 for setback requirements in the mineral zone.

13.  The impervious surface area for any lot may be increased beyond the total amount permitted in this chapter subject to approval of a conditional use permit.

14.  Required on property lines adjoining residential zones except when a stand-alone townhouse development on property designated commercial outside of center in the urban area is proposed to be located adjacent to property upon which an existing townhouse development is located.

SECTION 17.  Ordinance 10870, Section 343, as amended, and K.C.C. 21A.12.060 are each hereby amended to read as follows:

Minimum urban residential density.  Minimum density for residential development in the urban areas designated by the comprehensive plan shall be based on the tables in K.C.C. 21A.12.030, adjusted as provided for in 21A.12.070 - .080.

A.  A proposal may be phased, when compliance with the minimum density requirement results in noncompliance with the standards of K.C.C. chapter 21A.28, provided the overall density of the proposal is consistent with this section.

B.  Minimum density requirements may be waived by King County if the applicant demonstrates one or more of the following:

1.  The proposed layout of the lots in a subdivision or the buildings in a multiple dwelling development will not preclude future residential development consistent with the minimum density of the zone.

2.  The non-sensitive area of the parcel is of a size or configuration that results in lots which cannot meet the minimum dimensional requirements of the zone.

3.  In the R-12 through R-48 zones, the area of the parcel required to accommodate storm water facilities exceeds ((10)) ten percent of the area of the site.

4.  The site contains a national, state or county historic landmark.

C.  A proposal to locate a single residential unit on a lot shall be exempt from the minimum density requirement provided the applicant either pre-plans the site by demonstrating that the proposed single residence would be located in a manner compatible with future division of the site in a manner which would meet the minimum density requirements, or locates the dwelling within fifteen feet of one or more of the site's interior lot lines.

D.  Alternative minimum density requirements may be imposed through county-approved property-specific development standards (P-Suffix) or special district overlays (SDO) pursuant to K.C.C. chapter 21A.38.

SECTION 18.  Ordinance 10870, Section 344, as amended, and K.C.C. 21A.12.070 are each hereby amended as follows:

Calculations - allowable dwelling units, lots or floor area.  Permitted number of units, lots or floor area shall be determined as follows:

A.  The allowed number of dwelling units or lots (base density) shall be computed by multiplying the site area specified in K.C.C. 21A.12.080 by the applicable residential base density number;

B.  The maximum density (unit or lot) limits shall be computed by adding the bonus or transfer units authorized by K.C.C. chapters 21A.34 ((or)), 21A.36 or 21A.55 to the base units computed under subsection A;

C.  The allowed floor area, which excludes structured or underground parking area and areas housing mechanical equipment, shall be computed by applying the floor-to-lot area ratio to the project site area specified in K.C.C. 21A.12.080; and

D.  When calculations result in a fraction, the fraction shall be rounded to the nearest whole number as follows, except as provided in subsection E:

1.  Fractions of .50 or above shall be rounded up; and

2.  Fractions below .50 shall be rounded down.

E.  For subdivisions and short subdivisions in the RA, A and F zones, rounding up of the number of development units or lots shall not be allowed.

SECTION 19.  Ordinance 10870, Section 345, as amended, and K.C.C. 21A.12.080 are each hereby amended as follows:

Calculations – ((S))site area used for base density and maximum density floor area calculations.  A.  All site areas may be used in the calculation of base and maximum allowed residential density or project floor area except as outlined under the provisions of subsection B of this section.

B.  Submerged lands shall not be credited toward base and maximum density or floor area calculations.

C.  For subdivisions and short subdivisions in the RA zone, when calculations of site area for base density result in a fraction, the fraction shall be rounded to the nearest whole number as follows:

1.  Fractions of .50 or above shall be rounded up; and

2.  Fractions below .50 shall be rounded down.

SECTION 20.  Ordinance 10870, Section 351, as amended, and K.C.C. 21A.12.140 are each hereby amended to read as follows:

Setbacks – from regional utility corridors.  A.  In subdivisions and short subdivisions, areas used as regional utility corridors shall be contained in separate tracts.

B.  In other types of land development permits, easements shall be used to delineate such corridors.

C.  All ((buildings and)) structures shall maintain a minimum distance of five feet from property or easement lines delineating the boundary of regional utility corridors, except for utility structures necessary to the operation of the utility corridors.

D.  Any structure designed for human occupancy shall maintain a minimum distance of one hundred feet from a hazardous materials pipeline located within a regional utility corridor.  The setback distance may be modified where the applicant demonstrates the following:

1.  A one-hundred-foot setback would deny all reasonable use of the property; or

2.  That the structure would be protected from radiant heat of an explosion by berming or other physical barriers; or

3.  That a one-hundred-foot setback would be impractical or unnecessary due to existing geographical features, streets, lot lines, or easements; and

4.  That the applicant will construct a hazardous materials containment system or other mitigating facility if the county finds that leakage could accumulate within one hundred feet of the pipeline.  Any containment system or other mitigating facility required by this section shall meet all applicable federal, state and local regulations.

SECTION 21.  Ordinance 10870, Section 357, as amended, and K.C.C. 21A.12.200 are each hereby amended to read as follows:

Lot or site divided by zone boundary.  When a lot or site is divided by a zone boundary, the following ((rules shall apply)) applies:

A.  When a lot or site contains both residential and non-residential zoning, the zone boundary between the zones shall be considered a lot line for determining permitted building height and required setbacks on the site((;)).

B.  When a lot or site contains residential zones of varying density((,)):

1.  Any residential density transfer within the lot or site shall be allowed ((from the portion with the lesser residential density to that of the greater residential density;

2.  Residential density transfer from the higher density zone to the lower density zone may be allowed only)) when:

a.  ((the units transferred from any R-12 to R-48 zoned portion of the lot are maintained in an attached dwelling unit configuration on the lower density portion receiving such units,)) the density, as a result of moving dwelling units from one lot to another lot within a site or across zone lines within a single lot, does not exceed one hundred fifty percent of the base density on any of the lots or portions of a lot to which the density is transferred,

b.  the transfer does not reduce the minimum density achievable on the lot or site,

c.  the transfer enhances the efficient use of needed infrastructure,

d.  the transfer does not result in significant adverse impacts to the low density portion of the lot or site,

e.  the transfer contributes to preservation of environmentally sensitive areas, wildlife corridors, or other natural features, and

f.  the transfer does not result in significant adverse impacts to adjoining lower density properties.

2.  Residential density transfers from one lot to another lot within a site or from one portion of a lot to another portion of a lot across a zone line shall not be allowed in the rural residential area.

3.  Compliance with these criteria shall be evaluated during review of any development proposals in which such a transfer is proposed((; and)).

C.  Uses on each portion of the lot shall only be those permitted in each zone pursuant to K.C.C. chapter 21A.08.

SECTION 22.  Ordinance 10870, Section 359, as amended, and K.C.C. 21A.12.220 are each hereby amended to read as follows:

Nonresidential land uses in residential zones.  Except for utility facilities, uses listed in K.C.C. 21A.08.100, and nonresidential uses regulated by 21A.12.230, all nonresidential uses located in the RA, UR, or R zones shall be subject to the following requirements:

A.  Impervious surface coverage shall not exceed:

1.  Forty percent of the site in the RA zone.

2.  Seventy percent of the site in the UR and the R-1 through R-8 zones.

3.  Eighty percent of the site in the R-12 through R-48 zones.

B.  Buildings and structures, except fences and wire or mesh backstops, shall not be closer than ((30)) thirty feet to any property line, except as provided in subsection C.  In the RA zone, when more than one building or structure is located on a site, the first building or structure constructed should meet the street and interior setbacks set forth in K.C.C. 21A.12.030.  Additional buildings or structures should have varying setbacks, provided that additional buildings or structures, except fences, shall be no closer than thirty feet from any property line.

C.  Single detached dwelling allowed as accessory to a church or school shall conform to the setback requirements of the zone.

D.  Except for in the RA zone, ((P))parking areas are permitted within the required setback area from property lines, provided such parking areas are located outside of the required landscape area.

E.  Sites shall abut or be accessible from at least one public street functioning at a level consistent with King County Road Design Standards.  New high school sites shall abut or be accessible from a public street functioning as an arterial per the King County Design Standards.

F.  The base height shall conform to the zone in which the use is located.  The base height for nonresidential uses in the RA zone may not be exceeded by increasing the street or interior setbacks, as provided in K.C.C. 21A.12.030B.4, except for fire training towers, hose drying towers and nonresidential uses in the RA zone which require a Special Use Permit.

G.  Each individual building for nonresidential uses in the RA zone shall not exceed ten thousand square feet of gross floor area and the total gross floor area of all buildings for nonresidential uses in the RA zone on a single site shall not exceed twenty thousand square feet except as follows:

1.  Building square footage for those uses requiring a Special Use Permit shall be determined through the permit review process;

2.  The following uses are exempt from the square foot limitations of this section:

a.  stable;

b.  farm product warehousing, refrigeration and storage;

c.  livestock sales;

d.  all resource land uses permitted in the RA zone pursuant to K.C.C. 21A.08.090;

e.  public schools as defined by RCW 28A.150.010, except not including school district support facility as defined in K.C.C. 21A.06.1030; and

f.  zoo animal breeding facility.

H.  The building square footage limitations set forth in subsection G. do not apply to resuse of existing facilities or to primary or accessory uses related to agriculture or forestry when a farm or forestry management plan has been approved by King County department of natural resources.

((G.)) I.  Building illumination and lighted signs shall be designed so that no direct rays of light are projected into neighboring residences or onto any street right-of-way.

NEW SECTION.  SECTION 23.  There is hereby added to K.C.C. chapter 21A.12 a new section to read as follows:

Lot area – minimum lot size in rural area.  A.  Lots created through a subdivision or short subdivision in the RA zone shall have a minimum lot size equal to seventy-five percent of the zoning density.  For example, in the RA-5 zone the minimum lot size shall be seventy-five percent of five acres, or three and three-quarters acres.

SECTION 24.  Ordinance 10870, Section 364, as amended, and K.C.C. 21A.14.040 are each hereby amended to read as follows:

Lot segregations – ((C))clustered development.  When residential lot clustering is proposed, the following provisions shall be met:

A.  Any open space resulting from lot clustering shall not be altered or disturbed except as specified on recorded documents creating the open space.  Such open spaces may be retained under ownership by the subdivider, conveyed to residents of the development, or conveyed to a third party.  When access to the open space is provided, the access shall be located in a separate tract;

B.  In the RA zone:

1.  No more than eight lots of less than ((2.5)) two and one half acres shall be allowed in a cluster;

2.  No more than eight lots of less than ((2.5)) two and one half acres shall be served by a single cul-de-sac street;

3.  Clusters containing two or more lots of less than ((2.5)) two and one half acres, whether in the same or adjacent developments, shall be separated from similar clusters by at least ((120)) one hundred twenty feet;

4.  The overall amount, and the individual degree of clustering shall be limited to a level that can be adequately served by rural facilities and services, including, but not limited to, on-site sewage disposal systems and rural roadways; ((and))

5.  A fifty-foot Type I landscaping screen, as defined in K.C.C. 21A.16.040, shall be provided along the frontage of all public roads.  The planting materials shall consist of species that are native to the Puget Sound region.  Preservation of existing healthy vegetation is encouraged and may be used to augment new plantings to meet the requirements of this section;

6.  Open space tracts created by clustering in the RA zone shall be designated as permanent open space.  Acceptable uses within open space tracts are passive recreation, with no development of facilities, and natural-surface pedestrian and equestrian foot trails; and

7.  In the RA zone a resource land tract may be created through a cluster development in lieu of an open space tract.  The resource land tract may be used as a working forest or farm if the following provisions are met:

a.  Appropriateness of the tract for forestry or agriculture shall be determined by the King County department of natural resources;

b.  The subdivider shall prepare a forest management plan, which must be reviewed and approved by the King County department of natural resources or a farm management (conservation) plan, which must be developed by the King Conservation District.  The criteria for a forest management plan or farm management (conservation) plan prepared for a resource land tract established through a cluster development in the RA zone shall be set forth in a Public Rule.  The criteria must assure that forestry or farming will remain as a sustainable use of the resource land tract.  The plan must also set impervious surface limitations and identify they type of buildings or structures that will be allowed within the resource land tract;

c.  Clearing of forest cover shall not be permitted within the resource land tract in order to convert it to agricultural use;

d.  The recorded plat or short plat shall designate the resource land tract as a working forest or farm;

e.  Resource land tracts that are conveyed to residents of the development shall be retained in undivided interest by the residents of the subdivision or short subdivision;

f.  A homeowners association shall be established to assure implementation of the forest management plan or farm management (conservation) plan if the resource land tract is retained in undivided interest by the residents of the subdivision or short subdivision;

g.  The subdivider shall file a notice with the King County records and elections division.  The required contents and form of the notice shall be set forth in administrative rules.  The notice shall inform the property owner(s) that the resource land tract is designated as a working forest or farm, which must be managed in accordance with the provisions established in the approved forest management plan or farm management (conservation) plan;

h.  The subdivider shall provide proof of the approval of the forest management plan or farm management (conservation) plan and the filing of the notice with the records and elections division prior to the approval of the short plat or recording of the final plat; and

i.  The notice shall run with the land.

C.  In the R-1 zone, open space tracts created by clustering required by K.C.C. 21A.12.030 shall be located and configured to create urban separators and greenbelts as required by the comprehensive plan, ((community plans, or local)) or subarea plans or open space functional plans, to connect and increase protective buffers for environmentally sensitive areas as defined in K.C.C. 21A.06.1065, to connect and protect wildlife habitat corridors designated by the comprehensive plan, and to connect existing or planned public parks or trails.  King County may require open space tracts created under this subsection to be dedicated to an appropriate managing public agency or qualifying private entity such as a nature conservancy.  In the absence of such a requirement, open space tracts shall be retained in undivided interest by the residents of the subdivision or short subdivision.  A homeowners association shall be established for maintenance of the open space tract.

NEW SECTION.  SECTION 25.  There is hereby added to K.C.C. chapter 21A.14 a new section to read as follows:

Mixed use development – design features.  Mixed use development as defined in K.C.C. chapter 21A.06 and permitted by K.C.C. chapter 21A.08 shall incorporate the following design features:

A.  Residential and non-residential uses proposed for mixed use development shall be only those uses permitted in the zone, as established by K.C.C. chapter 21A.08.  At least one non-residential use in each mixed use development shall include a public gathering area when a sufficient amount of retail space is provided in the mixed use development to draw large groups of people.

B.  Where residential and nonresidential uses are proposed for the same structure, nonresidential uses shall occupy the lower levels and residential uses shall occupy the upper levels.  The director may waive this requirement under the following circumstances:

1.  When the structure is located on a sloping lot that provides exiting from upper levels or from multiple levels.  In such cases, the nonresidential use may be located on the levels that exit onto the primary pedestrian streets; or

2.  When views from the upper levels are valuable amenities that would help assure success of the nonresidential uses, such as a restaurant.

C.  Where residential and non-residential uses are proposed to occupy separate structures on a single site, the department shall not issue a temporary certificate of occupancy for the residential structure until the director approves a schedule for completion of the non-residential structure and the framing of the exterior walls of the non-residential structure has been inspected and approved.

D.  Non-residential uses such as artworks, parks, trails, arboretums or other outdoor recreational areas proposed as part of the mixed use development shall be established on the same site as the mixed use development and shall be easily accessible to the public.

E.  Mixed use development shall provide off-street parking behind or to the side of buildings, or enclosed within buildings consistent with K.C.C. 21A.18.030.  Relief from this provision may be granted by the director only if the applicant can demonstrate that there is no practical site design to meet this requirement.  The director may allow only the number of parking spaces that cannot be accommodated to the rear or sides of buildings, or enclosed within buildings, to be located to the front of buildings.  A twenty percent reduction of required parking is allowed when mixed use development meets the criteria of K.C.C. 21A.18.040 for shared parking.

SECTION 26.  Ordinance 10870, Section 374, and K.C.C. 21A.14.140 are each hereby repealed:

((Mixed use development - Shared parking.  For mixed use developments, a 20 percent reduction of required parking shall be permitted when the criteria of K.C.C. 21A.18.040. for shared parking facilities are met.))

NEW SECTION.  SECTION 27.  There is hereby added to K.C.C. chapter 21A.14 a new section to read as follows:

Rural nonresidential development standards.  A.  The purpose of this section is to establish standards for nonresidential development in rural areas which are outside of Rural Towns.  Site and building designs, buffering, tree and pasture land retention and compatibility of nonresidential uses are required to maintain rural character.

B.  The following development standards shall apply to nonresidential uses locating within the rural area:

1.  Storage of commodities sold at retail on the premises or materials used in fabrication of commodities shall be screened from adjoining rural residential uses.

2.  The landscaping standards set forth in K.C.C. chapter 21A.16 are modified as follows:

a.  a twenty-foot wide Type II landscaping, consisting of native vegetation, shall be provided along exterior streets;

b.  a twenty-foot wide Type I landscaping, consisting of native vegetation, shall be provided along property lines adjacent to rural residential zoned areas; and

c.  use of existing vegetation is encouraged to achieve the landscaping standard.

3.  On forested sites the following standards apply:

a.  clearing shall be the minimal amount required to accommodate the use;

b.  structures shall be sited so they have minimal visibility from the road; and

c.  forest undercover shall be maintained on forested portions of the site.

4.  On sites with pastures and meadows:

a.  open space shall be preserved as much as feasible; and

b.  buildings shall be located near existing vegetation and tree stands that border the open space.

5.  When more than one building is constructed, there shall be variation in the setbacks and orientation of the buildings, rather than uniformity, when locating the buildings on site.

6.  Outdoor lighting shall be focused downward and configured to minimize intrusion of light into surrounding rural residential areas.

7.  Refuse collection/recycling areas and loading or delivery areas shall be located at least one hundred feet from residential areas and screened with a solid view obscuring barrier.

8.  Off street parking standards shall be provided as follows:

a.  no less than one space for every one thousand square feet of floor area and no greater than one space for every five hundred square feet of floor area; and

b.  parking surface areas may be gravel rather than paved, however paving shall be on the portion of the parking area where it connects to the paved road.

9.  Signs are allowed as follows:

a.  signs shall not exceed an area of sixty-four square feet per sign;

b.  pole signs shall not be permitted; and

c.  signs shall not be internally illuminated.

C.  Building permit and land use permit applications shall include photographs of the site and an accompanying site plan map indicating the location and direction the photo were taken.  The photos shall include views onto the site from the public roads and views from the site to adjacent properties.  The photos shall be dated.

D.  Nonresidential uses in the rural area shall comply with the density and dimension standards set forth in K.C.C. 21A.12.220 for nonresidential land uses in residential zones.

E.  The standards established in this section. shall not apply to uses requiring a Special Use Permit or to the siting of public schools.

NEW SECTION.  SECTION 28.  There is hereby added to K.C.C. chapter 21A.14 a new section to read as follows:

Hazardous material pipelines.  A.  Tracts and easements containing hazardous materials pipelines and required setbacks from such pipelines may include the following uses, subject to other regulations applicable to each use and approval of the holder of the easement:  utility structures necessary for the operation of the pipeline, landscaping, trails, open space, keeping of animals, agriculture, forestry, commercial signage, minor communication facilities, and other compatible uses as specified on the face of the recorded plat or short plat; provided that structures designed for human occupancy shall never be allowed within pipeline tracts, easements or setbacks.

B.  Hazardous materials pipelines shall not be located in aquifer recharge areas, landslide hazard areas or erosion hazard areas.

SECTION 29.  Ordinance 10870, Section 378, as amended, and K.C.C. 21A.14.180 are each hereby amended to read as follows:

On-site recreation – ((S))space required.  A.  Residential developments ((if)) of more than four units in the UR and R-4 through R-48 zones, stand-alone townhouse developments in the NB zone on property designated commercial outside of center in the urban area of more than four units, and mixed use developments ((if)) of more than four units, shall provide recreation space for leisure, play and sport activities as follows:

1.  Residential subdivision, ((and)) townhouses and apartments developed at a density of eight units or less per acre - ((390)) three hundred ninety square feet per unit;

2.  Mobile home park - ((260)) two hundred sixty square feet per unit; and

3.  Apartment, townhouses developed at a density of greater than eight units per acre, and mixed use:

a.  Studio and one bedroom - ((90)) ninety square feet per unit;

b.  Two bedroom - ((130)) one hundred thirty square feet per unit; and

c.  Three or more bedroom - ((170)) one hundred seventy square feet per unit.

B.  Recreation space shall be placed in a designated recreation space tract if part of a subdivision.  The tract shall be dedicated to a homeowner’s association or other workable organization acceptable to the director, to provide continued maintenance of the recreation space tract consistent with K.C.C. 21A.14.200.

C.  Any recreation space located outdoors which is not part of a storm water tract developed in accordance with subsection F. shall:

1.  Be of a grade and surface suitable for recreation improvements and have a maximum grade of five percent;

2.  Be on the site of the proposed development;

3. Be located in an area where the topography, soils, hydrology and other physical characteristics are of such quality as to create a flat, dry, obstacle-free space in a configuration which allows for passive and active recreation;

4. Be centrally located with good visibility of the site from roads and sidewalks;

((3))5.  Have no dimensions less than ((20)) thirty feet (except trail segments);

6.  Be located in one designated area, unless the director determines that residents of large subdivisions, townhouses and apartment developments would be better served by multiple areas developed with recreation or play facilities;

((4))7.  In single detached or townhouse subdivisions, when the required outdoor recreation space exceeds ((5000)) five thousand square feet, the space shall have a street roadway or parking area frontage along ((10)) ten percent or more of the recreation space perimeter (except trail segments) when the outdoor recreation space is located in a single detached or townhouse subdivision;

((5)) 8.  Be accessible and convenient to all residents within the development; and

((6)) 9.  Be located adjacent to, and be accessible by trail or walkway to any existing or planned municipal, county or regional ((community)) park, public open space or trail system, which may be located on adjoining property.

((C)) D.  Indoor recreation areas may be credited towards the total recreation space requirement, when the ((county)) director determines that such areas are located, designed and improved in a manner which provides recreational opportunities functionally equivalent to those recreational opportunities available outdoors. For senior citizen assisted housing, indoor recreation areas need not be functionally equivalent but may include social areas, game and craft rooms, and other multi-purpose entertainment and education areas.

E.  Play equipment or age appropriate facilities shall be provided within dedicated recreation space areas according to the following requirements:

1.  For developments of five dwelling units or more, a tot lot or children’s play area, which includes age appropriate play equipment and benches, shall be provided consistent with K.C.C. 21A.14.190;

2.  For developments of five to twenty-five dwelling units, one of the following recreation facilities shall be provided in addition to the tot lot or children’s play area:

a.  playground equipment;

b.  sport court;

c.  sport field;

d.  tennis court; or

e.  any other recreation facility proposed by the applicant and approved by the director.

3.  For developments of twenty-six to fifty dwelling units, at least two or more of the recreation facilities listed in E.2. above shall be provided in addition to the tot lot or children’s play area;

4.  For developments of more than fifty dwelling units, one or more of the recreation facilities listed in E.2 above shall also be provided for every twenty-five dwelling units in addition to the tot lot or children’s play area.  When calculations result in a fraction, the fraction shall be rounded to the nearest whole number as follows:

a.  Fractions of .50 or above shall be rounded up; and

b.  Fractions below .50 shall be rounded down.

((D))F.  In subdivisions, recreation areas which are contained within the onsite ((S))stormwater ((runoff)) tracts, but are located outside of the one hundred year design water surface, may be credited for up to ((50%)) fifty percent of the required square footage of the on-site recreation space requirement on a foot-per-foot basis, subject to the following criteria:

1.  ((The stormwater runoff tract is dedicated or reserved as a part of a recreation space tract)) The stormwater tract and any onsite recreation tract shall be contiguously located.  At final plat recording, contiguous stormwater and recreation tracts shall be recorded as one tract and dedicated to the homeowner’s association or other organization as approved by the director;

2.  The ((detention pond)) stormwater facilities shall be constructed to meet the following conditions:

a.  The side slope of the stormwater facilities shall not exceed ((33%)) thirty three percent unless slopes are existing, natural and covered with vegetation;

b.  A bypass system or an emergency overflow pathway shall be designed to handle flow exceeding the facility design and located so that it does not pass through active recreation areas or present a safety hazard;

c.  The stormwater facilities shall be landscaped and developed ((in a manner to enhance)) for passive recreation opportunities such as trails, picnic areas and aesthetic viewing; and

d.  The stormwater facilities shall be designed so they do not require fencing pursuant to the Surface Water Design Manual.

((  3.))G.  In the case of joint use of the tract for stormwater facilities and recreation space, ((the)) King County ((department of public works)) shall be responsible for maintenance of the stormwater facilities only and will require ((an access)) a drainage easement for that purpose.

H.  A recreation space plan shall be submitted to the department and reviewed and approved with engineering plans.

1.  The recreation space plans shall address all portions of the site which will be used to meet recreation space requirements of this section (including stormwater facilities).  The plans shall show dimensions, finished grade, equipment, landscaping and improvements, as required by the director, to demonstrate that the requirements of the on-site recreation space per K.C.C. 21A.14.180 and play areas per K.C.C. 21A.14.190 have been met.

2.  If engineering plans indicate that the onsite stormwater facilities or stormwater tract must be increased in size from that shown in preliminary approvals, the recreation plans must show how the required minimum recreation space pursuant to K.C.C. 21A.14.180(A) will be met.

SECTION 30.  Ordinance 11621, Section 49, and K.C.C. 21A.14.185 are each hereby amended to read as follows:

Recreation space – ((F))fees in lieu of.  A.  The creation of on-site recreation space shall be the preferred method of providing new development with opportunities for leisure, play and sports activities.  Applicants shall to the best of their ability endeavor to provide such space on the project site.  However, ((I))if on-site recreation space is not provided pursuant to K.C.C. 21A.14.180, the applicant shall pay a fee-in-lieu of actual recreation space if approved by King County.  King County acceptance of ((this)) a fee-in-lieu payment is discretionary((, and may be permitted, if the proposed on-site recreation space does not meet the criteria of this chapter, or)).  Fee-in-lieu of onsite recreation space may be permitted if the recreation space provided within a county park in the vicinity will be of greater benefit to the prospective residents of the development.

B.  Fees ((provided in-lieu of on-site recreation space)) shall be determined annually by the department of parks and recreation ((division)) on the basis of the ((typical))projected market value of the required recreation space land area ((prior to the))after development.  Any recreational space provided by the applicant shall be credited toward the required fees.

C.  Payment of a fee-in-lieu does not exempt the applicant from providing minimum play area requirements per K.C.C. 21A.14.190.

D.  If recreation space credit is applied to stormwater facilities pursuant to K.C.C. 21A.14.180E, the development loses its option to request a fee-in-lieu and the remainder of the required recreation space and play area must be provided on site.

SECTION 31.  Ordinance 10870, Section 379, as amended, and K.C.C. 21A.14.190 are each hereby amended to read as follows:

On-site recreation -  ((P))play areas required.  A.  All single detached subdivisions, apartment, townhouse and mixed use developments, of more than four units in the UR and R-4 through R-48 zones and stand-alone townhouse developments in the NB zone of more than four units on property designated commercial outside of center in the urban area, excluding age restricted senior citizen housing, shall provide ((to)) children play areas within the recreation space on-site, except when facilities are available to the public ((within 1/4 mile)) that ((are developed as parks or playgrounds and are accessible without crossing of arterial streets.)) meet all of the following requirements:

1.  Developed as a county, municipal or regional park;

2.  Are located within one quarter mile walking distance; and

3.  Are accessible without crossing any arterial street.

B.  ((If any play apparatus is provided in the play area, the apparatus shall meet Consumer Product Safety Standards for equipment, soft surfacing and spacing, and shall be located in an area that is)) Play area designs shall comply with the following requirements:

1.  ((At least 400 square feet in size with no dimension less than twenty feet; and)) Provide at least forty five square feet per dwelling unit, with a minimum size of four hundred square feet;

2.  Be ((A))adjacent to main pedestrian paths or near building entrances((.));

3.  Meet the requirements of K.C.C. 21A.14.180; and

4.  Provide play equipment that meets , at a minimum, the Consumer Product Safety Standards for equipment, soft surfacing and spacing.

SECTION 32.  Ordinance 10870, Section 380,as amended, and K.C.C. 21A.14.200 are each hereby amended to read as follows:

On-site recreation – ((M))maintenance of recreation space or dedication.  A.  Recreation space as defined in  K.C.C. 21A.14.180B may, at the discretion of the department of parks and recreation, be dedicated as a park open to the public in lieu of providing the on-site recreation required ((above)) pursuant to K.C.C. 21A.14.180 when the following criteria are met:

1.  The dedicated area is at least twenty acres in size, except when adjacent to an existing or planned county park;

2.  The dedicated land provides one or more of the following:

a.  Shoreline access,

b.  Regional trail linkages,

c.  Habitat linkages,

d.  Recreation facilities, or

e.  Heritage sites;  and

3.  The dedicated area is located within 1 mile of the project site.

B.  Unless the recreation space is dedicated to King County pursuant to subsection A., maintenance of any recreation space retained in private ownership shall be the responsibility of the owner or other separate entity capable of long-term maintenance and operation in a manner acceptable to the department ((parks division)).

NEW SECTION.  SECTION 33.  There is hereby added to K.C.C. chapter 21A.14 a new section to read as follows:

Financial guarantees.  Financial guarantees for construction of recreation facilities required by K.C.C. 21A.14.180 and .190 shall be provided consistent with the provisions of K.C.C Title 27A.

SECTION 34.  Ordinance 10870, Section 388, as amended, and K.C.C. 21A.16.030 are each hereby amended to read as follows:

Land use grouping.  In order to facilitate the application of this chapter, the land uses of K.C.C. chapter 21A.08 have been grouped in the following manner.

A.  Residential development shall refer to those uses listed in K.C.C. 21A.08.030, except those uses listed under Accessory uses, provided:

1.  Attached/group residences shall refer to:

a.  townhouses, except as provided in subsection 2a;

b.  apartments, and detached dwelling units developed on common property at a density of ((12)) twelve or more units per acre;

c.  senior citizen assisted housing;

d.  temporary lodging;

e.  group residences other than Type I community residential facilities;

f.  mobile home parks; and

2.  Single family development shall refer to:

a.  residential subdivisions and short subdivisions, including attached and detached dwelling units on individually platted or short platted lots;

b.  any detached dwelling units located on a lot; and

c.  Type I community residential facilities.

B.  Commercial development shall refer to those uses in:

1.  K.C.C. 21A.08.040 as amusement/entertainment uses, except golf facilities;

2.  K.C.C. 21A.08.050 except recycling centers, health and educational services, daycare I, churches, synagogues, and temples; and

3.  K.C.C. 21A.08.070, except forest product sales and agricultural crop sales.

C.  Industrial development shall refer to those uses listed in:

1.  K.C.C. 21A.08.050 as recycling center;

2.  K.C.C. 21A.08.060 except government services;

3.  K.C.C. 21A.08.080; and

4.  K.C.C. 21A.08.090 as mineral extraction and processing.

D.  Institutional development shall refer to those uses listed in:

1.  K.C.C. 21A.08.040 as cultural uses, except arboretums;

2.  K.C.C. 21A.08.050 as churches, synagogues and temples, health services, and education services except specialized instruction schools permitted as an accessory use; and

3.  K.C.C. 21A.08.060 as government services.

E.  Utility development shall refer to those uses listed in K.C.C. 21A.08.060 as utility facilities.

F.  Uses contained in K.C.C. chapter 21A.08 that are not listed in subsections A-E of this section shall not be subject to landscaping and tree retention requirements except as specified in any applicable review of a conditional use or special use permits.

SECTION 35.  Ordinance 10870, Section 390, as amended, and K.C.C. 21A.16.050 are each hereby amended to read as follows:

Landscaping – ((S))street frontages.  The average width of perimeter landscaping along street frontages shall be provided as follows:

A.  Twenty feet of Type II landscaping shall be provided for an institutional use, excluding playgrounds and playfields;

B.  Ten feet of Type II landscaping shall be provided for an industrial development;

C.  Ten feet of Type II landscaping shall be provided for an above ground utility facilities development, excluding distribution and transmission corridors, located outside a public right-of-way;

D.  Ten feet of Type III landscaping shall be provided for a commercial or attached/group residence development; and

E.  For single family subdivisions and short subdivisions:

1.  Trees shall be planted at the rate of one tree for every ((40)) forty feet of frontage along ((a neighborhood collector street or arterial)) all public streets.

2.  The trees shall be:

a.  Located within the street right-of-way if permitted by the custodial state or local agency;

b.  No more than ((20)) twenty feet from the street right-of-way line when located within a lot;

c.  Maintained by the adjacent landowner unless part of a county maintenance program; and

d.  A species approved by the county if located within the street right-of way and compatible with overhead utility lines.

3.  The trees may be spaced at irregular intervals in order to accommodate sight distance requirements for driveways and intersections.

SECTION 36.  Ordinance 10870, Section 392, as amended, and K.C.C. 21A.16.070 are each hereby amended to read as follows:

Landscaping – ((S))surface parking areas. Parking area landscaping shall be provided within surface parking areas with ten or more parking stalls for the purpose of improving air quality, reducing surface water runoff, providing shade and diminishing the visual impacts of large paved areas as follows:

A.  Residential developments with common parking areas shall provide  planting areas at the rate of ((20)) twenty square feet per parking stall;

B.  Commercial, industrial, or institutional developments, shall provide landscaping at a  rate of:

1.  Twenty square feet per parking stall when ((10)) ten to ((30)) thirty parking stalls are provided; and

2.  Twenty-five square feet per parking stall when ((31)) thirty-one or more parking stalls are provided;

C.  Trees shall be provided and distributed throughout the parking area at a rate of:

1.  One tree for every ((five)) three parking stalls for a commercial or industrial development; and

2.  One tree for every ((ten)) five parking stalls for residential or institutional development;

D.  The maximum distance between any parking stall and landscaping shall be no more than ((100)) one hundred feet;

E.  Permanent curbs or structural barriers shall be provided to protect the plantings from vehicle overhang; ((and))

F.  Landscaping around the perimeter of a site that is in addition to the perimeter landscaping required by K.C.C. 21A.16.050 may count toward ten percent of the required surface parking area landscaping if it is adjacent to the parking area; and

G.  Parking area landscaping shall consist of:

1.  Canopy-type deciduous trees, evergreen trees, evergreen shrubs and ground covers planted in islands or strips;

2.  Shrubs that do not exceed a maintained height of ((42)) forty-two inches;

3.  Plantings contained in planting islands or strips having an area of at least ((100)) one hundred square feet and with a narrow dimension of no less than five feet;

4.  Ground cover pursuant to K.C.C. 21A.16.090; and

5.  At least ((70)) seventy percent of trees are deciduous.

SECTION 37.  Ordinance 10870, Section 415, as amended, and K.C.C. 21A.18.110 are each hereby amended to read as follows:

Off-street parking plan design standards.  A. Off-street parking areas shall not be located more than ((600)) six hundred feet from the building they are required to serve for all uses except those specified below; where the off-street parking areas do not abut the buildings  they serve, the required maximum distance shall be measured from the nearest building entrance that the parking area serves:

1.  For all single detached dwellings the parking spaces shall be located on the same lot they are required to serve;

2.  For all other residential dwellings at least a portion of parking areas shall be located within ((150)) one hundred fifty feet from the building(s) they are required to serve;

3.  For all non-residential uses permitted in residential zones, the parking spaces shall be located on the same lot they are required to serve and at least a portion of parking areas shall be located within ((150)) one hundred fifty feet from the nearest building entrance they are required to serve;

4.  In designated activity, community business, and neighborhood business centers, parking lots ((should)) shall be located to the rear or sides of buildings ((, when feasible)).  Relief from this provision may be granted by the director only if the applicant can demonstrate that there is no practical site design to meet this requirement.  The director may allow only the number of parking spaces that cannot be accommodated to the rear or sides of buildings to be located to the front of buildings;

5.  Parking lots shall be so arranged as to permit the internal circulation of vehicles between parking aisles without re-entering adjoining public streets; and

6.  Parking for the disabled shall be provided in accordance with K.C.C. 21A.18.060.

B.  The minimum parking space and aisle dimensions for the most common parking angles are shown on the chart below.  For parking angles other than those shown on the chart, the minimum parking space and aisle dimensions shall be determined by the director.  Regardless of the parking angle, one-way aisles shall be at least ten feet wide, and two-way aisles shall be at least twenty feet wide.  If dead-end aisles are used in the parking layout, they shall be constructed as two-way aisles.  Parking plans for angle parking shall use space widths no less than eight feet six inches for a standard parking space design and eight feet for a compact car parking space design.

MINIMUM PARKING STALL AND AISLE DIMENSIONS.*

| A |  | B |  |  | C |  |  | D |  | E |  |  | F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PARKING ANGLE |  |  | STALL WIDTH |  |  | CURB LENGTH |  |  | STALL DEPTH |  | AISLE WIDTH 1-WAY 2-WAY |  |  | UNIT DEPTH 1-WAY 2-WAY |
| 0 0 |  |  | 8.0* Min 8.5 Desired 9.0 |  |  | 20.0* 22.5 22.5 |  |  | 8.0 8.5 9.0 |  | 12.0 20.0 12.0 20.0 12.0 20.0 |  |  | ** ** 29.0 37.0 30.0 38.0 |
| 30 30 |  |  | 8.0* Min 8.5 Desired 9.0 |  |  | 16.0* 17.0 18.0 |  |  | 15.0 16.5 17.0 |  | 10.0 20.0 10.0 20.0 10.0 20.0 |  |  | ** ** 42.0 53.0 44.0 54.0 |
| 45 45 |  |  | 8.0* Min 8.5 Desired 9.0 |  |  | 11.5* 12.0 12.5 |  |  | 17.0* |  | 12.0 20.0 12.0 20.0 12.0 20.0 |  |  | ** ** 50.0 58.0 51.0 59.0 |
| 60 60 |  |  | 8.0* Min 8.5 Desired 9.0 |  |  | 9.6* 10.0 10.5 |  |  | 18.0 20.0 21.0 |  | 18.0 20.0 18.0 20.0 18.0 20.0 |  |  | ** ** 58.0 60.0 60.0 62.0 |
| 90 | 8.0* Min 8.5 Desired 9.0 |  |  | 8.0* 8.5 9.0 |  |  | 16.0* 18.0 18.0 |  |  | 24.0 24.0 24.0 24.0 23.0 24.0 |  | ** ** 60.0 60.0 60.0 60.0 |  |  |

* for compact stalls only

** variable with compact and standard combinations

*

C.  Any parking spaces abutting a required landscaped area on the driver or passenger side of the vehicle shall provide an additional ((18)) eighteen inches above the minimum space width requirement to provide a place to step other than in the landscaped area.  The additional width shall be separated from the adjacent parking space by a parking space division stripe.

D.  The parking space depth may be reduced when vehicles overhang a walkway or landscaping under the following conditions:

1.  Wheelstops or curbs are installed;

2.  The remaining walkway provides a minimum of forty-eight inches of unimpeded passageway for pedestrians;

3.  The amount of space depth reduction is limited to a maximum of eighteen inches; and

4.  Landscaping is designed in accordance with K.C.C. 21A.16.070.E.

E.  Driveways providing ingress and egress between off-street parking areas and abutting streets shall be designed, located and constructed in accordance with the provisions of K.C.C. chapter 14.42, Road Standards.  Driveways for single detached dwellings, no more than twenty feet in width, may cross required setbacks or landscaped areas in order to provide access between the off-street parking areas and the street, provided no more than fifteen percent of the required landscaping or setback area is eliminated by the driveway.  Joint use driveways may be located within required landscaping or setback areas.  Driveways for all other developments may cross or be located within required setbacks or landscaped areas in order to provide access between the off-street parking areas and the street, provided no more than ten percent of the required landscaping is displaced by the driveway and the driveway is located no closer than five feet from any property line except where intersecting the street.

F.  Parking spaces required per this title shall be located as follows:

1.  For single detached dwelling units the required parking spaces shall be outside of any required setbacks or landscaping, provided driveways crossing setbacks and required landscaping may be used for parking.  However, if the driveway is a joint use driveway, no vehicle parked on the driveway shall obstruct any joint user’s access to the driveway or parking spaces;

2.  For all other developments parking spaces may be permitted by the director in setback areas pursuant to an approved landscape plan; and

3.  For non-residential uses in residential zones, parking is permitted in setback areas pursuant to K.C.C. 21A.12.220.

G.  Lighting shall be provided for safety of traffic and pedestrian circulation on the site.  It shall be designed to minimize direct illumination of abutting properties and adjacent streets.  The director shall have the authority to waive the requirement to provide lighting.

H.  Tandem or end-to-end parking is allowed in residential developments.  Apartment/townhouse developments may have tandem parking areas for each dwelling unit but shall not combine parking for separate dwelling units in tandem parking areas.

I.  All vehicle parking and storage for single detached dwellings must be in a garage, carport or on an approved impervious surface.  Any impervious surface used for vehicle parking or storage must have direct and unobstructed driveway access.

J.  The total number of vehicles parked or stored outside of a building on a single family lot in the R-4 through R-8 zones, excluding recreational vehicles and trailers, shall not exceed six vehicles on lots ((12,500)) twelve thousand five hundred square feet or less and eight vehicles on lots greater than ((12,500)) twelve thousand five hundred square feet.

K.  Vanpool/carpool parking areas shall meet the following minimum design standards:

1.  A minimum vertical clearance of seven feet three inches shall be provided to accommodate van vehicles if designated vanpool/carpool parking spaces are located in a parking structure; and

2.  A minimum turning radius of twenty-six feet four inches with a minimum turning diameter (curb to curb) of fifty-two feet five inches shall be provided from parking aisles to adjacent carpool/vanpool parking spaces.

L.  Direct access from the street right-of-way to off-street parking areas shall be subject to the requirements of K.C.C. 21A.28.120.

M.  No dead-end alley may provide access to more than eight off-street parking spaces.

N.  Any parking stalls located in enclosed buildings must be totally within the enclosed building.

SECTION 38.  Ordinance 10870, Section 481, as amended, and K.C.C. 21A.24.340 are each hereby amended to read as follows:

Wetlands: ((M))mitigation requirements.  A.  Restoration shall be required when a wetland or its buffer is altered in violation of law or without any specific permission or approval by King County.  The following minimum requirements shall be met for the restoration of a wetland:

1.  the original wetland configuration shall be replicated including its depth, width, length and gradient at the original location;

2.  the original soil type and configuration shall be replicated;

3.  the wetland edge and buffer configuration shall be restored to its original condition;

4.  the wetland, edge and buffer shall be replanted with vegetation native to King County which replicates the original vegetation in species, sizes and densities; and

5.  the original wetland functions shall be restored including, but not limited to, hydrologic and biologic functions.

B.  The requirements in subsection A. may be modified if the applicant demonstrates that greater wetland functions can otherwise be obtained.

C.  Replacement shall be required when a buffer is altered pursuant to an approved development proposal or a wetland is used for a regional flow control facility or other approved use.  The requirements for the restoration of wetlands shall be met by replacement wetlands.

D.  Enhancement may be allowed when a wetland or buffer will be altered pursuant to a development proposal, but the wetland’s biologic and/or hydrologic functions will be improved.  Minimum requirements for enhancement shall be established in administrative rules.

E.  All alterations of wetlands shall be replaced or enhanced on the site or within the same drainage basin using the following formulas:  class 1 and 2 wetlands on a 2:1 basis and class 3 wetlands on a 1:1 basis with equivalent or greater  biologic functions including, but not limited to, habitat functions and with equivalent hydrologic functions including, but not limited to, storage capacity.

F.  Replacement or enhancement off the site may be allowed if the applicant demonstrates to the satisfaction of King County that the off-site location is in the same drainage sub-basin as the original wetland and that greater biologic and hydrologic functions will be achieved.  The formulas in subsection E. shall apply to replacement and enhancement off the site.

G.  Surface water management or flood control alterations including, but not limited to, wetponds shall not constitute replacement or enhancement unless other functions are simultaneously improved.

H.  Mitigation sites should be located to alleviate wildlife habitat fragmentation, and avoid impacts to and prevent loss of farmable land within agricultural production districts.

SECTION 39.  Ordinance 11621, Section 72, and K.C.C. 21A.24.345 are each hereby amended to read as follows:

Wetlands: ((M))mitigation banking.  King County may consider and approve replacement or enhancement of unavoidable adverse impacts to wetlands caused by the development activities through an approved wetland mitigation bank.  Wetland mitigation banking is not allowed in the agricultural production districts when the purpose is to compensate for filling wetlands for development outside of the agricultural production districts.  Compensatory mitigation in advance of authorized impacts must be provided through an approved mitigation bank.  Criteria governing the creation and use of a mitigation bank shall be established in administrative rules.  A pilot project or projects, complete with evaluation should be initiated which would test the viability of the mitigation bank concept prior to its full implementation.

SECTION 40.  Ordinance 11168, Section 5, as amended, and K.C.C. 21A.30.062 are each hereby amended to read as follows:

Animal regulations – ((L))livestock – ((B))building requirements.  A.  In residential zones, fee boarding of livestock other than in a legally established stable shall only be as an accessory use to a resident on the subject property; and

B.  A barn or stable may contain a caretaker’s accessory living quarters under the following conditions:

1.  Only one accessory living quarter per primary detached dwelling unit, except in the F zone where accessory living quarters are not allowed;

2.  The accessory living quarter shall not exceed five hundred square feet, and

3.  The structure must be constructed in conformance with the State Building Code.

SECTION 41.  Ordinance 10870, Section 563, and K.C.C. 21A.34.040 are each hereby amended to read as follows:

Public benefits and density incentives.  A.  The public benefits eligible to earn increased densities, and the maximum incentive to be earned by each benefit, are set forth in subsection ((E.))F. The density incentive is expressed as additional bonus dwelling units (or fractions of dwelling units) earned per amount of public benefit provided.

B.  Bonus dwelling units may be earned through any combination of the listed public benefits.

C.  The guidelines for affordable housing bonuses including the establishment of rental levels, housing prices and asset limitations, will be updated and adopted annually by the council in the ((comprehensive housing affordability strategy))consolidated housing and community development plan.

D.  Bonus dwelling units may also be earned and transferred to the project site through the transfer of density credit (“TDC”) process set forth in K.C.C. chapters 21A.36 or 21A.55, by providing any of the open space, park site or historic preservation public benefits set forth in subsections ((E))F.2 or ((E))F.3 on sites other than that of the RDI development.

E.  Residential development in R-4 through R-48 zones with property specific development standards requiring any public benefit enumerated in this chapter, shall be eligible to earn bonus dwelling units as set forth in subsection ((E))F. when the public benefits provided exceed the basic development standards of this title.  When a development is located in a special overlay district, bonus units may be earned if the development provides public benefits exceeding corresponding standards of the special district.

F.  The following are the public benefits eligible to earn density incentives through RDI review:

| BENEFIT |  | DENSITY INCENTIVE |
| --- | --- | --- |
| 1. AFFORDABLE HOUSING |  |  |
| a. Benefit units consisting of rental housing permanently priced to serve non-senior citizen low-income households (i.e. no greater than 30 percent of gross income for households at or below 50 percent of King County median income, adjusted for household size). A covenant on the site that specifies the income level being served, rent levels and requirements for reporting to King County shall be recorded at final approval. |  | 1.5 bonus units per benefit unit, up to a maximum of 30 low-income units per five acres of site area; projects on sites of less than five acres shall be limited to 30 low-income units. |
| b. Benefit units consisting of rental housing designed and permanently priced to serve low-income senior citizens (i.e. no greater than 30 percent of gross income for 1 or 2-person households, 1 member of which is 62 years of age or older, with incomes at or below 50 percent of King County median income, adjusted for household size). A covenant on the site that specifies the income level being served, rent levels and requirements for reporting to King County shall be recorded at final approval. |  | 1.5 bonus units per benefit unit, up to a maximum of 60 low-income units per five acres of site area; projects on sites of less than five acres shall be limited to 60 low-income units. |
| c. Benefit units consisting of senior citizen assisted housing units 600 square feet or less. |  | 1 bonus unit per benefit unit |
| d. Benefit units consisting of moderate income housing reserved for income- and asset-qualified home buyers (total household income at or below 80 percent of King County median, adjusted for household size). Benefit units shall be limited to owner-occupied housing with prices restricted based on typical underwriting ratios and other lending standards, and with no restriction placed on resale. Final approval conditions shall specify requirements for reporting to King County on both buyer eligibility and housing prices. |  | .75 bonus unit per benefit unit. |
| e. Benefit units consisting of moderate income housing reserved for income- and asset-qualified home buyers (total household income at or below 80 percent of King County median, adjusted for household size). Benefit units shall be limited to owner-occupied housing with prices restricted based on typical underwriting ratios and other lending standards, and with a 15 year restriction binding prices and eligibility on resale to qualified moderate income purchasers. Final approval conditions shall specify requirements for reporting to King County on both buyer eligibility and housing prices. |  | 1 bonus unit per benefit unit. |
| f. Benefit units consisting of moderate income housing reserved for income- and asset-qualified home buyers (total household income at or below 80 percent of King County median, adjusted for household size). Benefit units shall be limited to owner-occupied housing, with prices restricted to same income group, based on current underwriting ratios and other lending standards for 30 years from date of first sale. A covenant on the site that specifies the income level and other aspects of buyer eligibility, price levels and requirements for reporting to King County shall be recorded at final approval. |  | 1.5 bonus units per benefit unit. |
| g. Projects in which 100 percent of the units are reserved for moderate income - and asset-qualified buyers (total household income at or below 80 percent of the King County median, adjusted for household size). All units shall be limited to owner-occupied housing with prices restricted based on current underwriting ratios and other lending standards, and with prices restricted to same income group, for 15 years from date of first sale. Final approval conditions shall specify requirements for reporting to King County on both buyer eligibility and housing prices. |  | 200 percent of the base density of the underlying zone. Limited to parcels 5 acres or less in size and located in the R-4 through R-8 zones. Housing types in the R-4 or R-6 zones shall be limited to structures containing four or less units, except for townhouses. Such RDI proposals shall not be eligible to utilize other RDI bonus density incentives listed in this section. |
| h. Benefit units consisting of mobile home park space or pad reserved for the relocation of an insignia or non-insignia mobile home, that has been or will be displaced due to closure of a mobile home park located in incorporated or unincorporated King County. |  | 1.0 bonus unit per benefit unit. |
| 2. OPEN SPACE, TRAILS AND PARKS |  |  |
| a. Dedication of park site or trail right-of-way meeting King County location and size standards for neighborhood, community or regional park, or trail, and accepted by the parks division. |  | .5 bonus unit per acre of park area or quarter-mile of trail exceeding the minimum requirement of K.C.C. 21A.14 for on-site recreation space or trail corridors, computed on the number of dwelling units permitted by the site’s base density. |
| b. Improvement of dedicated park site to King County standards for developed parks. |  | .75 bonus unit per acre of park improvement. If the applicant is dedicating the site of the improvements, the bonus units earned by improvements shall be added to the bonus units earned by the dedication. |
| c. Improvement of dedicated trail segment to King County standards. |  | 1.8 bonus units per quarter-mile of trail constructed to county standard for pedestrian trails; or |
|  |  | 2.5 bonus units per quarter-mile of constructed to county standard for multipurpose trails (pedestrian/ bicycle/equestrian). |
|  |  | Shorter segments shall be awarded bonus units on a pro-rate basis. If the applicant is dedicating the site of the improvements, the bonus units earned by improvements shall be added to the bonus units earned by the dedication. |
| d. Dedication of open space, meeting King County acquisition standards to the county or a qualified public or private organization such as a nature conservancy. |  | .5 bonus unit per acre of open space. |
| 3. HISTORIC PRESERVATION |  |  |
| a. Dedication of a site containing an historic landmark in accordance with K.C.C. 20.62, to King County or a qualifying nonprofit organization capable of restoring and/or maintaining the premises to standards set by the King County Landmarks Commission. |  | .5 bonus unit per acre of historic site. |
| b. Restoration of a site or structure designated as an historic landmark in accordance with K.C.C. 20.62 to a specific architectural or site plan approved by the King County Landmarks Commission. |  | .5 bonus unit per acre of site or one thousand square feet of floor area of building restored. |
| 4. ENERGY CONSERVATION |  |  |
| a. Benefit units that incorporate conservation features in the construction of all on-site dwelling units heated by electricity that save at least 20 percent of space heat energy use from the maximum permitted by the Northwest Energy Code, as amended. No more than 50 percent of the required savings may result from the installation of heat pumps. None of the required savings shall be achieved by reduction of glazing area below 15 percent of floor area. Energy use shall be expressed as allowable energy load per square foot or as total transmittance (UA). |  | 0.15 bonus unit per benefit unit that achieves the required savings. |
| b. Benefit units that incorporate conservation features in the construction of all on-site dwelling units heated by natural gas, or other non-electric heat source, that save at least 25 percent of space heat energy use from the maximum permitted by the Northwest Energy Code, as amended. None of the required savings shall be achieved by reduction of glazing area below 15 percent of floor area. Energy use shall be expressed as allowable energy load per square foot or as total transmittance (UA). |  | 0.10 bonus unit per benefit unit that achieves the required savings. |
| c. Developments located within 1/4 mile of transit routes served on at least a half-hourly basis during the peak hours and hourly during the daytime non-peak hours. |  | 10 percent increase above the base density of the zone. |
| 5. PUBLIC ART |  |  |
| a. Devoting 1 % of the project budget to public art on site. |  | 5 percent increase above the base density of the zone. |
| b. Contributing 1 % of the project budget to the King County public art fund for development of art projects. The contribution shall be used for projects located within a one mile radius of the development project. |  | 5 percent increase above the base density of the zone. |

NOTE:	When proposed energy conservation bonus units of K.C.C. 21A.34.040 are reviewed in conjunction with a subdivision or a short subdivision, the applicant shall provide data and calculations for a typical house of the type to be built in the development that demonstrates to the department's satisfaction how the required savings will be achieved.  A condition of approval shall be recorded with the plat and shown on the title of each lot specifying the required energy savings that must be achieved in the construction of the dwelling unit.  The plat notation shall also specify that the savings shall be based on the energy code in effect at the time of preliminary plat application.

SECTION 42.  Ordinance 12823, Section 19, and K.C.C. 21A.38.240 are each hereby repealed:

((Special district overlay - Floodplain Density.  A.  The purpose of the floodplain density special district overlay is to provide a means to designate areas that cannot accommodate additional density due to severe flooding problems.  This district overlay limits development in sensitive areas to reduce potential future flooding.

B.  The following development standards shall be applied to all development proposals on RA-5 zoned parcels located within a floodplain density special district overlay:

1.  Density is limited to one home per 10 acres for any property that is located within a sensitive area; and

2.  All development shall be clustered outside of the identified sensitive areas, unless the entire parcel is a mapped sensitive area.))

SECTION 43.  Ordinance 10870, Section 634 (part), and K.C.C. 21A.42.140 are each hereby amended to read as follows:

Review process for high schools.  A.  The School District shall hold a public hearing on the request for a building permit on the proposed high school and may merge the public hearing for environmental review with this hearing.  The hearing shall address the proposal's compliance with the applicable development standards and whether the impacts of traffic on the neighborhood have been addressed pursuant to the State Environmental Policy Act, Chapter 43.21C RCW, and/or through the payment of road impact fees.  The hearing may be conducted by the Board of Directors, or where authorized by board policy, by a hearing examiner appointed by the School Board.  The District shall provide notice of the hearing as follows:

1.  by posting the property;

2.  by publishing in a newspaper of general circulation in the general area where the proposed high school is located;

3.  by sending notices by first class mail to owners of property in an area within ((500)) five hundred feet of the proposed high school, provided such area shall be expanded as necessary to send mailed notices to at least ((20))twenty different property owners;  and

4.  by sending notices to other residents of the District that have requested notice.

B.  At a regularly scheduled or special Board meeting, the Board of Directors shall adopt findings of compliance with applicable King County development standards, including the decision criteria outlined in K.C.C. chapter 21A.44, or adopt proposed actions necessary to reach compliance.  Where a hearing examiner has been appointed, the Board of Directors shall review and adopt or reject the hearing examiner's proposed findings and/or proposed actions.  The board may include in the record any information supporting its findings or any information from prior public meetings held on the same general subject at the discretion of the Board.

C.  Copies of the findings and/or the proposed actions shall be mailed to all parties of record and to the county.

D.  Any aggrieved party of record may request the Board of Directors to reconsider the findings within ((20))twenty calendar days of its adoption.  An aggrieved party requesting reconsideration shall submit written evidence challenging the findings or otherwise specifically identify reasons why the District has failed to reasonably comply with the applicable King County development standards or the decision criteria outlined in K.C.C. chapter 21A.44.  Within ((30))thirty calendar days after a request for reconsideration has been filed with the District, the Board of Directors may reconsider and revise the findings and/or proposed actions, or may decline to reconsider.  Failure to act, or to initiate the process for reconsideration by notifying the aggrieved party of record of intent to reconsider, within the ((30-))thirty day period shall be deemed to constitute a decision not to reconsider.

E.  The Board's final findings shall be attached to the District's building permit application and shall be considered as prima facie evidence of compliance with the applicable King County development standards.

SECTION 44.  Ordinance 13274, Section 2, and K.C.C. 21A.55.110 are each hereby amended to read as follows:

Transfer of development credit (TDC) pilot program  - framework for monitoring.  This section establishes a framework for evaluating the effectiveness of the provisions of Ordinance 13274, as follows:

A.  Pilot projects shall be initiated by private parties within thirty-six  months of the effective date of Ordinance 13274 (10/5/98).

B.  Within the thirty-six month effective period for the TDC ((P))pilot ((P))program, the executive shall:

1.  Develop a means of monitoring and assessing the pilot program in order to improve future applications and to measure outcomes based on the TDC purposes described in K.C.C. 21A.55.100; and

2.  Submit annual fourth quarter reports to the council  which describe:

a.  the use of the pilot program in the preceding year,

b.  the public benefit associated with each sending site qualified under this ordinance, such as resource protection or preservation of a model forest in the rural forest focus areas ((district)),

c.  current receiving area market conditions, and recommendations for improved regulatory, fiscal and marketing strategies,

d.  community acceptance of the program and associated higher densities in the receiving areas, including recommendations for improved and/or flexible development standards to ensure quality urban and rural development, and

e.  opportunities for new sending and receiving areas, including cooperative planning with cities.

SECTION 45.  Ordinance 13274, Section 4, and K.C.C. 21A.55.130 are each hereby amended to read as follows:

Transfer of development credit (TDC) pilot program - sending sites.

A.  For the purpose of this chapter, a sending site is defined as the portion of the parcel(s) qualifying under subsection B below.  Sending sites shall be maintained in a natural state, except for agricultural lands, rural ((farm and)) forest ((district lands)) focus areas, proposed public park or trail sites suitable for active or passive recreation or historic sites.  Nonresidential uses consistent with the zone may be allowed following the transfer of residential development credits if allowed under the conservation easement.

B.  Qualification of a sending site shall demonstrate that the site contains a public benefit such that preservation of that benefit by transferring residential development density to another site is in the public interest.

1.  Designation in the King County Comprehensive Plan or a functional plan as ((A))agricultural land or as ((R))rural ((Farm or F))forest ((Districts))focus area land which is not encumbered through King County’s development rights purchase program; or

2.  Designation in the King County Comprehensive Plan, or a functional plan as a proposed park site, through either:

a.  designation of a specific site, or

b.  identification of geographic areas of proposed park sites which meet adopted standards and criteria; or

3.  A written determination by a public land managing agency, including but not limited to the King County department of parks and recreation, King County department of natural resources, King County office of cultural resources, a city parks department (if a sending site is located within that city's designated potential annexation area), the Washington State Parks and Recreation Commission or a private land conservation organization such as the Nature Conservancy, that the sending site is appropriate for preservation or acquisition as:

a.  open space,

b.  wildlife habitat for federal or state listed endangered or threatened species, priority or candidate priority species of local importance; or species of local significance as defined by the King County Comprehensive Plan; or wildlife networks designated by King County, or Priority Habitats as defined by the state,

c.  urban separators,

d.  regional trail/natural linkages, or

e.  historic landmarks.

The agency making the written determination of qualification shall state that the sending site is appropriate for preservation or acquisition under that agency’s rules, regulations or guidelines and shall state that the proposed form of permanent protection is acceptable to the agency responsible for managing the sending site once encumbered (i.e. encumbrance with a conservation easement or fee simple ownership).  Following the encumbrance or dedication of the sending site, the remaining land value may be acquired or accepted by the managing agency.

C  .For the purposes of the TDC ((P))pilot ((P))program, "acquisition" shall mean obtaining fee simple rights in real property, or a less than a fee simple right in a form which preserves in perpetuity the public benefit supporting the designation or qualification of the property as a sending site.

SECTION 46.  Ordinance 13274, Section 5, and K.C.C. 21A.55.140 are each hereby amended to read as follows:

Transfer of development credit (TDC) pilot program - receiving sites.

A.  Receiving sites shall be:

1.  Zoned R-4 through R-48, NB, CB, RB or O, or any combination thereof.  Such  sites may also be within potential annexation areas established pursuant to the countywide planning policies,

2.  King County incorporated municipal jurisdictions with urban centers (as designated under the countywide planning policies), transit station sites, and/or other urban areas where new growth is or will be encouraged under the Growth Management Act and the countywide planning policies and where facilities and services exist or where public investments in facilities and services will be made, or

3.  RA-2.5 and RA-5 zoned parcels which meet the criteria listed below may receive development credits transferred from ((R))rural ((Farm and F))forest ((Districts)) focus areas, and accordingly may be subdivided and developed at a maximum density of one dwelling per ((2.5)) two and a half acres.  Increased density allowed through the designation of rural receiving areas:

a.  must be eligible to be served by domestic Group A public water service;

b.  must be located within one-quarter mile of an existing predominant pattern of rural lots smaller than ((5)) five acres in size;

c.  must not adversely impact regionally or locally significant resource areas or environmentally sensitive areas;

d.  shall not require public services and facilities to be extended to create or encourage a new pattern of smaller lots; ((and))

e.  may not be located within ((R))rural ((Farm and)) ((F))forest ((Districts)) focus areas; and

f.  may not be located on Vashon-Maury Islands.

B.  Except as provided in this chapter development of an unincorporated King County receiving site shall remain subject to all zoning code provisions for the base zone.

C.  Apartments and townhouse units are permitted outright in the R-4 through R-8 zones through the transfer of density credits provisions of this pilot project, subject to the review process described in K.C.C. 21A.55.180.

D.  An unincorporated King County receiving site may accept development credits from one or more sending sites, up to the maximum density permitted pursuant to K.C.C. 21A.12.030.

SECTION 47.  Ordinance 13274, Section 6, and K.C.C. 21A.55.150 are each hereby amended to read as follows:

Transfer of development credit (TDC) pilot program - transfer rules.

A.  Unincorporated urban sending sites and unincorporated urban receiving sites.  The number of residential  development credits that an unincorporated urban sending site is eligible to send to an unincorporated urban receiving site shall be determined by:

1.  Applying  twenty-five percent of the sending site’s base zoned density to the sending site’s unbuildable sensitive areas for R-4 through R-48 zoned properties and one credit per acre for R-1 zoned properties.  Due to the limitations imposed by K.C.C. chapter 21A.24, for the purposes of this chapter unbuildable sensitive areas shall include class 1 and class 2 wetlands, streams, slopes forty percent or steeper and associated buffers; and

2.  applying the base density of the zone the sending site is located in to the remaining portion of the sending site less the following:

a.  any portion of the sending site already in a conservation easement or other similar encumbrance,

b.  any land area already used to calculate residential density for other development within the sending site,

c.  any submerged land,

d.  regional utility corridors; and

e.  other areas, excluding setbacks, required by King County to remain undeveloped.

Any fractions of development credits which result from the calculations in K.C.C. 21A.55.150A1 or A2 above shall not be included in the final determination of total development credits available for transfer.

3.  The twenty-five percent discount for unbuildable sensitive areas in K.C.C. 21A.55.150A1 above shall not be applied to urban sending sites certified using the process prescribed in this chapter prior to December 31, 1998.

B.  Rural sending sites and unincorporated urban receiving sites.  For purposes of the transfer of development credit (TDC) program, property within rural forest focus areas shall have a base density of one dwelling unit per five acres.  The number of residential development credits that a rural area sending site is eligible to send to an urban area receiving site shall be determined by applying twice the base density of the zone in which the rural area sending site is located, after the following lands have been subtracted:

1.  Any portion of the sending site already in a conservation easement or other similar encumbrance,

2.  Any land area already used to calculate residential density for other development within the sending site, and

3.  Any submerged lands.

Any fractions of development credits which result from the calculations above shall not be included in the final determination of total development credits available for transfer.

C.  Rural sending sites and incorporated urban receiving sites.  For purposes of the transfer of development credit (TDC) program, property within rural forest focus areas shall have a base density of one dwelling unit per five acres.  The number of development credits that a rural area sending site is eligible to send to an incorporated urban area receiving site shall be determined through the application of a conversion ratio established by the incorporated municipal jurisdiction.  The conversion ratio will be applied to the number of  available sending site credits determined by applying  the base density of the zone in which the sending site is located, after the following lands have been subtracted:

1.  Any portion of the sending site already in a conservation easement or other similar  encumbrance,

2.  Any land area already used to calculate residential density for other development within the sending site, and

3.  Any submerged lands.

Any fractions of development credits which result from the calculations above shall not be included in the final determination of total development credits available for transfer.

D.  Rural sending sites and rural receiving sites.  For purposes of the transfer of development credit (TDC) program, property within rural forest focus areas shall have a base density of one dwelling unit per five acres.  The number of residential development credits that a sending site is eligible to send to a receiving site shall be determined by applying the base density of the zone in which the sending site is located, after the following lands have been subtracted:

1.  Any portion of the sending site already in a conservation easement or other similar encumbrance,

2.  Any land area already used to calculate residential density for other development within the sending site, and

3.  Any submerged lands.

Any fractions of development credits which result from the calculations above shall not be included in the final determination of total development credits available for transfer.

E.  Following the transfer of residential development credits from either rural or urban sending sites, the portion of the parcel(s) not designated as a sending site may accommodate a lot or lots on the buildable portion of the parcel(s), consistent with the zoned base density provisions of the density and dimensions table in K.C.C. 21A.12.030, the allowable dwelling unit calculations in K.C.C. 21A.12.070, and other King County development regulations.  In the case of sending sites within the rural area, the development potential remaining after a density transfer may only be actualized through a clustered subdivision, short subdivision or binding site plan that creates a permanent preservation tract as large or larger than the portion of the subdivision set aside as lots.  Within rural forest focus areas ((districts)), such preservation resource use tracts shall be at least fifteen acres in size.

F.  A site plan showing unbuildable sensitive areas and buffers as defined in this section, submerged lands, regional utility corridors, areas required by King County to remain undeveloped and conservation easements or other similar encumbrances shall be submitted as part of the sending site certification application.

G.  Development credits from one sending site may be allocated to more than one receiving site and one receiving site may accept development credits from more than one sending site.

H.  Following the transfer of credits from a sending site, deed restrictions documenting the development credit transfers shall be recorded and notice placed on the title to the sending site parcel.

I.  A conservation easement granted to the county or other appropriate land  management agency shall be required for land contained in the sending site.  The conservation easement shall be documented by a map indicating the portion of the parcel restricted from future residential development, whether  or not such land is dedicated, as follows:

1.  A conservation easement, which contains the easement map, shall be recorded on the sending site to indicate development limitations on the sending site;

2.  For a sending site zoned A-10 or A-35, the conservation easement shall be consistent in form and substance with the purchase agreements used in the agricultural land development rights purchase program.  The conservation easement shall preclude subdivision of the subject property but may permit not more than one dwelling per sending site, and shall permit agricultural uses as provided in the A-10 or A-35 zone;

3.  For a sending site located within a rural forest focus area, the sending site shall be a minimum of twenty acres. ((district, t))The conservation easement shall require that fifteen acres of the sending site be restricted to forest management activities and shall include a forest stewardship plan approved by the county for ongoing forest management practices. ((encumber a minimum of fifteen acres, shall allow n))No more than one dwelling unit, calculated at a base density of one dwelling unit per five acres, is allowed for every ((per)) twenty acres.  The dwelling unit is to remain with the unrestricted portion of the conservation easement or unencumbered portion of the sending site ((and shall include a forest stewardship plan approved by the county for ongoing management practices)).

J.  Upon submitting an application to the department of development and environmental services to develop a receiving site under the provisions of this chapter, the receiving site applicant shall provide either a sending site certification letter and evidence of the option to buy the sending site development credits required for the receiving site development proposals or evidence of ownership or full legal control of all sending sites proposed to be used in calculating total residential density on the receiving site.

K.  Development credits from a sending site shall be considered transferred to a receiving site when a final decision is made on the TDC receiving area development proposal by the department of development and environmental services, the sending site is permanently protected by a completed and recorded land dedication or conservation easement, and notification has been provided to the King County assessor’s office.

L.  The determination of the number of residential development credits a sending site has available for transfer to a receiving site development shall be valid for transfer purposes only, shall be documented in a TDC certificate letter of intent and be considered a final determination, not to be revised due to changes to the sending site’s zoning.

M.  TDC receiving site developments shall comply with dimensional standards of the zone with a base density most closely comparable to the total approved density of the TDC receiving site development.

SECTION 48.  Severability.  If any provision of this ordinance or its application to any person or circumstance is held invalid, the remainder of the ordinance or the application of the provision to other persons or circumstances is not affected.

SECTION  49.  Pursuant to King County Code section 20.44.080, the Metropolitan King County Council finds that the requirements for environmental analysis, protections and mitigations in those chapters of King County Code Title 21A amended by this ordinance provide adequate analysis of and mitigation for the specific adverse environmental impacts to which the requirements apply.

Attachments:  None