ANALYSIS OF KEY POLICIES

2.1:  GENERAL POLICIES

The general policies form a “mission statement” providing guidance for all of its floodplain management activities and define the standard that is binding on cities, towns and special districts in King County.

Policy G-2 Flood Risks

The natural processes of flooding and channel migration become risks when human development is located within flood hazard areas. The level of risk is evaluated on a case by case basis using the predicted likelihood of flooding and channel migration and the consequences that would result if no action is taken. Flood risks, and the resulting consequences that would result if no action is taken, are generally prioritized in the following order:

- Threats to public safety.

- Damage to public infrastructure.

- Impacts on the regional economy.

- Damage to private structures.

Policy G-3: Comprehensive River and Flood Hazard Management

King County should provide comprehensive river and flood hazard management through the implementation of projects and programs that result in multiple benefits, including those created by meeting any or all of the following non-prioritized objectives:

- Effectively meet site- and reach-specific flood risk reduction needs;

- Achieve benefits that exceed the total cost of projects or programs, including long-term maintenance costs;

- Avoid the creation of new flood, channel migration or other risks that cannot be mitigated;

- Protect productive agricultural soils;

- Protect and, where possible, enhance aquatic and riparian habitat in a manner consistent with adopted salmon habitat recovery plans, and

- Leverage flood hazard management revenues through partnerships with other agencies and stakeholders.

Policy G-10: Protecting Natural Functions and Values

King County shall protect flood storage, conveyance, and ecological values of floodplains, wetlands, and riparian corridors and, when feasible, should enhance or restore these ecological functions and values.  Flood risk reduction strategies and projects should be coordinated on a river-reach scale with the salmon habitat recovery plans.

Analysis

Flooding and channel migration are natural processes. Undeveloped floodplains provide storage for floodwaters, open space, recreational opportunities and habitat for fish and wildlife. When development takes place in the floodplain, flooding and channel migration processes present risks to the development and are in turn affected by the development. Flooding and channel migration can result in damage to many types of public and private property. Spending public funds may be more appropriate for some types of properties and areas of risk than others. For example, County facilities such as roads, bridges, and parks represent a taxpayer investment and are an important part of the public infrastructure.

Watersheds do not follow jurisdictional boundaries. Actions taken by a city or county in one part of a watershed—whether it be a land-use plan, development permit, or capital improvement project—can increase flood and channel migration risks experienced by other jurisdictions in the watershed and can impact ecological resources. Actions taken by a city or county can also have positive effects on neighboring jurisdictions. Cooperative floodplain management among counties, cities, towns and special districts is supported by Chapter 86.12 RCW. Multi-jurisdictional approaches to watershed management can produce a multitude of public and private benefits, including flood risk reduction and improved ecosystem functions and values.

The King County Comprehensive Plan establishes King County government as both a regional and local service provider. Among the regional services King County provides are flood warning and floodplain management, as specified in King County Comprehensive Plan Policy F-103. Floodplain management includes a range of services at both the regional and local level to reduce the risk of flood and channel migration hazards.

Policy G-14: National Flood Insurance Program

King County shall exceed the federal minimum standards stipulated by the National Flood Insurance Program for unincorporated areas, utilizing the Community Rating System as a guide to better protect public safety, reduce the risk of flood and channel migration hazards to existing public and private property, and achieve flood insurance premium discounts.

Analysis

The National Flood Insurance Program provides a financial mechanism to respond to flood disasters by making flood insurance available to private property owners. This program provides federally-backed flood insurance that encourages communities to enact and enforce floodplain regulations in order to qualify residents for flood insurance. Communities can also receive flood insurance premium discounts by adopting regulations that exceed federal minimum standards.

2.2:  FLOODPLAIN LAND USE POLICIES

These policies guide land-use planning and development regulations in floodplains

and channel migration hazard areas. The policies seek to reduce flood risks to future developments and prevent increased risks to surrounding properties using existing floodplain restrictions and do not require additional limitations.

Development in the floodplain can create two types of challenges:

- Because of its location in a hazardous area, the development may be at risk from inundation, erosion, or both.

- The development can increase risks to neighboring properties by creating a barrier to the conveyance of floodwaters, thus causing backwater flooding upstream, and by reducing the area available to store and slowly release floodwaters, increasing flow velocities and erosion downstream.

Policy FP-1: Future Conditions

Wherever future-conditions flows have been modeled and adopted or approved by King County as a component of a basin plan or technical study, they shall be used within unincorporated King County to define the 100-year future-conditions floodplain that would occur with build out in the basin under current zoning and regulations. In these basins, land use policies and flood hazard regulations shall apply to the 100-year future-conditions floodplain.

Analysis

Historically, King County flood hazard regulations have been applied within the 100-year floodplain as mapped by FEMA. FEMA maps are based on current or historical land use in the watershed. As watersheds develop, however, the rate and volume of runoff reaching rivers and streams can increase.  The boundaries of the 100-year floodplain may change over time, creating inconsistencies between actual floodplain conditions and those portrayed on FEMA maps. In addition, some rivers in King County migrate laterally, endangering properties along their banks. Areas that are at risk due to channel migration are sometimes outside the mapped floodplain, so that residents may not be aware of the risk.

2.3:  STRUCTURAL AND NON-STRUCTURAL PROJECTS POLICIES

The policies in this section guide a comprehensive program that can implement a range of flood hazard management projects, including both structural and non-structural projects. Structural projects consist primarily of revetments, levees, and structures associated with levees such as pump plants and flap gates. King County rarely pursues new construction of these types of structural projects; the majority of King County’s flood hazard management projects focus on maintenance and repair of structures built in the 1960s and 1970s. Maintenance and repair often involve retrofitting existing flood protection facilities, including setting back levees and replacing traditional riprap using biostabilization techniques for bank protection to reduce the long-term maintenance costs and provide habitat for fish and wildlife. Nonstructural projects include relocation, acquisition, and elevation of flood-prone homes and the removal of existing flood hazard management structures that are no longer needed.

Policy PROJ-1: Prioritizing Flood Hazard Risks

King County should prioritize actions to address flood and channel migration risks using the following criteria in order of importance:

- The consequences that will result if no action is taken. Consequences should be prioritized as identified in Policy G-2.

- Urgency, where urgency is a measure of how quickly an action needs to be taken in order to prevent a risk from growing worse.

- Legal responsibility and authority, where legal responsibility and authority is a contractual relationship between King County and another person or agency to maintain a flood protection facility.

- Funding or partnership opportunities.

Analysis

Hundreds of public and private properties in King County are vulnerable to flood-related hazards. If no action is taken to address a flooding or channel migration risk, King County needs to assess the consequences that will result. Some risks may need to be addressed sooner than others depending on the severity of the risk and what is vulnerable. Under certain circumstances, King County may have legal responsibility to take action to address flooding risks, such as when there is a contractual agreement with a property owner or another agency to maintain a flood protection facility.

Policy PROJ-2: Property Acquisition

Property acquisition for flood risk reduction projects should be voluntary on the part of the property owner, except under very limited circumstances.

Policy PROJ-3: Easements

New or additional easements necessary to construct, maintain, repair, or retrofit a flood protection facility should be sufficient to meet applicable King County design and construction standards and federal and state technical guidelines. Acquisition of easements for flood hazard management projects should be consistent with Policy PROJ-2.

Analysis

Structural and non-structural approaches to flood hazard management often necessitate the acquisition of property interests. When a flood protection facility such as a levee or revetment is constructed, maintained, repaired or reconstructed, sufficient land is needed to meet design standards, provide room for equipment and materials, and ensure that King County employees have access to the facility for monitoring and maintenance. In some cases, property acquisition takes the form of an easement granted to King County by the underlying property owner.

In other cases, King County may purchase land for flood hazard management purposes. King County is authorized under RCW 86.12.030 to condemn property for flood risk reduction purposes. King County would only exercise that authority under extreme circumstances, such as when federal, state or local regulations prohibit reconstruction on the site, development on the site is causing significant flood or erosion risks to other properties, or the property is needed to complete a flood risk reduction project.

All condemnations for flood risk reduction purposes must be approved by the King County Council prior to filing with the court. When King County purchases land through either a voluntary sale or through condemnation, the property owner is given fair

market value based on an independent private-sector appraisal.

King County is committed to managing its public lands, including vacant floodplain properties, for a variety of public values and benefits, including fish and wildlife habitat, passive recreation, trails, or other purposes that do not create a new risk from flooding and channel migration. Property acquisition in support of flood hazard management outcomes is often funded through external funding sources, some of which have specific deed restrictions associated with future uses of the property. Proposed uses of county-owned property are evaluated on a case-by-case basis to determine the impact the public mayhave on flood protection facilities and natural resources, as well as overall public safety issues.

2.4:  RIVER CHANNEL MAINTENANCE POLICIES

River and stream channels are dynamic systems that naturally transport logs, sediments and other debris from their upper reaches to lower elevations. These materials are integral parts of the natural ecology and geomorphology of the stream, and are particularly vital in the life cycle of salmon and trout. However, in some circumstances, these materials can increase flooding or channel migration risks. For this reason, King County has sometimes relocated large woody debris and removed sediment from river channels.

This section recommends policies to direct future channel maintenance activities undertaken by King County. The policies seek a balance between resource concerns and the protection of public property and private structures. Routine maintenance of natural drainage systems is conducted using best management practices contained in the King County Regional Road Maintenance Guidelines, which were designed to be compatible with the Endangered Species Act and were approved by the National Oceanic and Atmospheric Administration’s National Marine Fisheries Service.

Policy RCM-1: When to Reposition or Relocate Large Woody Debris

Naturally occurring accumulations of large woody debris should be repositioned or relocated for flood hazard management purposes only if one or more of the flood and channel migration risks in Policy G-2 is present and all reasonable flood and channel migration risk reduction alternatives have been considered.

Policy RCM-2: How to Reposition and Relocate Large Woody Debris

Repositioning or relocation of naturally occurring large woody debris should be accomplished using the technique that results in the least disturbance to the river channel and riparian corridor. If repositioned or relocated, the wood should be put back into the river in a manner that does not create new flood or channel migration risks. If it is not practical or reasonable to return the large woody debris to the channel, it should be incorporated into an adjacent riparian corridor.

Analysis

King County responds to requests from the public and other agencies to remove, reposition or relocate fallen trees from rivers and streams. This response includes an initial site investigation to assess the channel dynamics and level of risk created by the woody debris. In some cases, it is appropriate for King County to take action to remove or alter large woody debris to reduce flood or channel migration risks.

Policy RCM-3: Gravel Removal

King County should remove gravel from rivers and streams for flood hazard management purposes only when:

- It can be demonstrated that gravel accumulation poses a flood risk, as defined in Policy G-2 of this Plan,

- Hydraulic and geomorphic studies conclude gravel removal has a long-term benefit of flood risk reduction,

- Biologic studies determine it does not, with appropriate mitigation, result in a net loss of ecological function,

- It is part of a comprehensive, long-term flood management strategy,

- It is consistent with the best available science, the provisions of this Plan, and state and federal guidelines and regulations, including the Endangered Species Act, and

- It is determined to be the best flood risk reduction alternative available, based on criteria in this Plan.

Analysis

The policy applies only to projects that King County sponsors or participates in.  One of the most common requests King County receives from the public is to conduct dredging within channels that have large accumulations of gravel. If King County is to conduct gravel removal, its effectiveness as a flood risk reduction strategy, its potential adverse impacts, and its priority relative to other King County projects must be considered. If gravel removal is done, the operation must be conducted in a way that does not damage existing King County flood protection facilities.

2.6:  FUNDING POLICIES

King County’s major funding sources for flood hazard management are the River Improvement Fund and the Green River Flood Control Zone District. Other funding sources include the Inter-County River Improvement Agreement levy, which funds flood risk reduction projects on the White and Puyallup Rivers through an agreement between King and Pierce Counties.

King County has also been successful in leveraging federal and state grant and disaster assistance funds. The policies in this section provide a framework for making decisions about how these funding sources are used for flood hazard management in King County.

Policy FF-1: Economic Impact Assessment

King County should complete economic impact assessments along major river systems to determine the economic impact of potential flood-related damages that may occur as a result of failure of King County managed flood protection facilities. These analyses should be consistent with the River and Floodplain Management Program goals and objectives and should be used to inform flood hazard management actions and to prioritize funding allocations.

Analysis

A flood risk assessment is the process of measuring the potential loss of life, personal injury, economic injury, and property damage resulting from a flood by assessing the vulnerability of people, buildings and infrastructure to the flood hazard. A risk assessment is considered an integral part of the information needed to determine and prioritize funding for flood protection facility capital improvement projects and maintenance programs. It is also a key element in identifying the benefits of a project as required under federal grant programs. The structural integrity of existing flood protection facilities, some of which were constructed many years ago, is not fully known.

Development conditions within the flood hazard areas have also changed over time. Due to limited resources, King County has not conducted a comprehensive assessment of its flood protection facility inventory, or the development protected by these facilities.

A King County Risk Assessment was performed for this Plan using geographic information system data, pursuant to the requirements of the Disaster Mitigation Act and the Community Rating System, and is included in Appendix C of the Plan. However, the detail of approach in this King County Risk Assessment differs from the comprehensive assessment of facilities suggested in this section.

Contemporary risk assessment methods can be used to determine the current level of risk associated with King County flood protection facilities and should be viewed as dynamic tools that can change as information and technology change.

Policy FF-2: Designated Emergency Fund Balance

King County should establish a designated emergency fund balance reserve to be used exclusively for flood emergency response. To the extent that new funding is approved, these funds should be used to leverage state and federal funds, but should also be sufficient to allow King County to respond to emergencies when state and federal funding is not readily available. The reserve allocation should be based on the projected costs for the local share of flood disaster recovery plus a cost contingency.

Policy FF-3: Regional Funding

New or expanded regional funding sources should be identified and secured to meet the need for

enhanced or expanded flood hazard management projects and programs to address flood hazard

management goals and objectives.

Policy FF-4: Pre-Disaster Mitigation Grant Program

King County should identify and prioritize eligible projects on an annual basis for submittal to the FEMA Pre-Disaster Mitigation Grant Program.

Analysis

When a major flood event occurs, local jurisdictions are generally required to match federal and state funds that are available for recovery and repair of damaged structures or the mitigation of flood damaged structures.  There are also pre-disaster funds available under federal programs that require a local cost share.

Currently King County does not have a dedicated emergency fund balance that is sufficient to

meet the demand that could result from a major flood disaster. Current funding levels in support of flood risk reduction are significantly inadequate; additional regional funding sources that are integrated to meet the multiple objective needs of flood risk reduction are needed.