| # Review of the Cascade Water Alliance Transmission and Supply Plan |  |  |
| --- | --- | --- |
| Service Area The Cascade Water Alliance (CWA) owns and operates a municipal water system that serves the eight member cities and special purpose districts in King County. CWA is a Joint Municipal Utility Services Authority, formed under Revised Code of Washington (RCW) 39.106.030, composed of the City of Bellevue, Covington Water District, City of Issaquah, City of Kirkland, City of Redmond, Sammamish Plateau Water and Sewer District, Skyway Water and Sewer District, and City of Tukwila. CWA is primarily located in central King County generally east and south of Lake Washington and in south King County. CWA’s purposes generally include contracting with other regional water suppliers on behalf of its members; developing and operating water supply facilities; providing regional water conservation services; and planning for the water needs of its members (including long-range and short-term plans for emergencies or water shortages). Water deliveries from CWA began on January 1, 2004. CWA delivers water to its members entirely on a “wholesale” basis. Each member is responsible for distributing water to its own residents, businesses, and other “retail” customers. Members own and operate their own local distribution systems for these purposes. Some members also have their own independent water supplies and meet all or a portion of their needs separately from the wholesale supply provided by CWA. CWA has no franchise with the County and will seek a franchise in the 2018-2019 time period if and when it constructs water transmission mains that propose to use County right of way. Each CWA member is responsible under state law to prepare a water system plan every six years. However, regional planning for water supply sources is delegated to CWA. In light of the requirements in chapter 246-290-100 of the Washington Administrative Code (WAC), this Transmission and Supply Plan (TSP) provides the following specific elements relevant to water system plans prepared by CWA members: - Description of current and future regional water supplies that are either delivered currently or planned for future delivery to CWA members; - Documentation of the water-use efficiency program administered by CWA on behalf of its members; - Source of supply analysis; - Documentation of CWA’s water rights; - Water shortage response plan as it relates to regional water supplied by CWA; - Discussion of opportunities for use of reclaimed water from regional sources such as King County; and - Discussion of capital improvements related to CWA water supplies and facilities. Reclaimed Water CWA neither owns nor operates a wastewater treatment plant (WTP). One member agency, the City of Tukwila, uses reclaimed water in its service area now. The Identification and Screening of Potential Supply Sources included reclaimed water as a potential source and evaluated it in a similar manner to other sources. - SEPA - CWA completed a State Environmental Policy Act (SEPA) checklist for the TSP and, as lead agency, issued a determination of non-significance for the issuance of the TSP on December 30, 2011. There were no appeals. |  |  |
| A review of the specific statutes, rules, codes, and policies to the water system plan is as follows: |  |  |
|  | A. General water plan, King County Code (KCC) 13.24.010 and 13.28 | Comments/findings |
| (1) | - Review is applicable to utilities that obtain or distribute water in unincorporated King County under KCC 13.24. - Is there a need to meet the consistency requirements of Revised Code of Washington (RCW) 43.20.260? | - The TSP supplements information on regional supply presented in each members’ individual water system plan. - Some members of CWA provide or distribute water in unincorporated King County and the TSP is subject to King County Council approval. - Yes, the Washington State Department of Health (DOH) also needs to determine consistency of the TSP under RCW 43.20.260. |
| (2) | - Consistency with King County Comprehensive Plan (KCCP) and development regulations, and policies including KCC 21A.28.040 development standards, provision of adequate supplies for anticipated growth and development. | - Yes, the TSP is consistent. |
| (3) | - Infrastructure for existing and future service areas based on adopted land use map. | - CWA's existing capital facilities for delivery of municipal water supplies consist solely of the Bellevue-Issaquah Pipeline. - Individual members of CWA own, operate, and maintain their own water distribution systems. This includes, as applicable, water treatment for their independent water sources; maintenance of water quality within their reservoirs and distribution piping systems; and local monitoring of water quality conditions - Five of the eight CWA members have their own independent supplies. |
| (4) | - Review proposals for modified or expanded service areas based on compliance with utility’s approved plan and ability to meet duty to serve requirement. | - The future service area is not proposed for change in the TSP. - CWA may deliver contracted supplies to other water purveyors in King and Pierce counties for use in their water service areas. - The duty to serve policies for retail service area do not apply to CWA. |
| (5) | - Sufficient information to demonstrate the ability to provide service consistent with the requirements of all applicable statutes, codes, rules, and regulations. | - Yes, the TSP demonstrates an ability to provide service consistent with applicable statutes, codes, and regulations. |
| (6) | - Monitor and review effectiveness of purveyor conservation plans if within area covered by an approved Coordinated Water System Plan (CWSP). | - CWA members are within three different Coordinated Water System Planning areas (CWSPs). The conservation efforts at the regional level assist the member systems in meeting the established policies, design standards, and goals that meet or exceed the requirements and goals of the CWSPs. |
|  | B. Consistency requirements: 13.24.060 |  |
| (7) | - State and local health requirements. | - Yes. |
| (8) | - Creation and maintenance of logical service areas. | - Yes, the service area is logical. |
| (9) | - Elimination or prevention of duplicate facilities. | - Yes. The CWA intends to provide service by means of wholesale water sales for its members. |
| (10) | - Promotion of most healthful and reliable services to the public. | - Yes. Water purchased by CWA complies with public health standards. - The water is chlorinated and fluoridated. |
| (11) | - Provision of service at a reasonable cost, and maximization of use of public facilities. | - Not applicable. |
| (12) | - KCCP and other pertinent county adopted plans and policies. | - Yes, there is consistency between the CWA TSP and the KCCP. |
| (13) | - Basin-wide or multibasin water plans, sewerage plans, or both when approved by the Washington State Department of Ecology (Ecology) or DOH. | - No multibasin water plans are applicable. - The TSP is consistent with the East King County CWSP, South King County CWSP, and Skyway CWSP. |
| (14) | - Applicable state water quality, water conservation, and waste management standards. | - Yes, applicable standards are met. Water purchased from Seattle Public Utility (SPU) is fluoridated and chlorinated. |
| (15) | - Water Resources Act (RCW 90.54). | - The TSP makes no connection with the requirements of RCW 90.54.180 or RCW 90.03.386. CWA will maintain its existing regional conservation program. - The eight member utilities will continue to comply individually with DOH’s Water Use Efficiency Rule. |
| (16) | - Growth Management Act (GMA) (RCW 36.70A). | - The TSP appropriately used King County land use zoning for that area of unincorporated King County served by the CWA. - The TSP used demographic and employment data from the Puget Sound Regional Council. - CWA's five member cities also have land use plans that are consistent with GMA. |
| (17) | - Ground water management plans. | - Not applicable at the regional level. - The individual member utility plans address ground water if they use ground water as a source of supply. |
| (18) | - Federally approved habitat conservation plans and recovery plans under the Endangered Species Act (ESA). | - Not discussed in TSP. |
| (19) | - Requirements for salmon recovery under Ch. 77.85 RCW, and other plans, including regional water supply or water resource management plans. | - Not discussed in the TSP. |
| (20) | - Applicable requirements to evaluate opportunities for the use of reclaimed water under chapter 90.46 RCW and CO-7. | - The TSP contains a detailed reclaimed water feasibility analysis as part of the supply portfolio analysis. |
|  | C. KCCP—consistency with provisions and specific policies (Water System Plan) |  |
|  | COUNTYWIDE PLANNING POLICIES |  |
| (21) | FW-5: management of resources for multiple beneficial uses, including flood and erosion hazard reduction. | - Not applicable. |
| (22) | FW-12(c): ensure sufficient water supply for growth and fish habitat needs through long-term planning. | - Sufficient water supply for projected growth is available. CWA uses surface water from the regional system as well as member-owned ground water sources. - Sufficient water for fish habitat is not addressed. |
| (23) | CA-5, CA-6, and E-434 and policies to protect quantity and quality of groundground waterwater. | - Not applicable. |
| (24) | CO-5: water supply shall be regionally coordinated. | - Yes. CWA contracts with SPU for delivery of water to seven of the eight CWA members. - The SPU supply contract provides for a “declining block” of supply that will be reduced in five-year increments beginning in 2024. - CWA conducted an extensive review of potential future supply options and has identified supply through 2030. |
| (25) | CO-6: aggressive conservation efforts shall be implemented. | - CWA adopted a modest conservation goal that relies upon member utilities implementing their conservation programs to meet the regional goal. |
|  | KING COUNTY COMPREHENSIVE PLAN POLICIES |  |
| (26) | F-102: King County will provide or manage countywide services, which include wastewater, water resource management, surface water management, flood warning and floodplain management, protection and preservation of natural resource lands. | - Yes, for that portion of the service area that is in unincorporated King County. |
| (27) | F-104: plan for provision of services to rural areas. | - CWA members’ service areas includes portions of the rural unincorporated area. - CWA and its members are committed to serving the area consistent with the KCC. |
| (28) | F-105: King County to work with cities and service providers to establish priority areas for public funding of capital facilities. | - The capital improvement program (CIP) is adequate and appropriately focused with funding sources identified. - CWA anticipates a Tacoma-Cascade Pipeline will be needed in 2024. This transmission link will convey water supply contracted from Tacoma and the Covington Water District north to other members. Various configurations are possible for the Tacoma-Cascade Pipeline. The assumed pipeline diameter is 30 inches. - CWA also anticipates a storage reservoir will be needed by 2024 to balance deliveries through the system. |
| (29) | F-201: all facilities and services should be provided in compliance with provisions and requirements of the ESA. | - Not discussed in Plan. |
| (30) | F-202: ensure adequate supply of public facilities to support communities. | - Yes, CWA has sufficient water supplies. |
| (31) | F-203: King County will work with cities, special purpose districts, and other service providers to define regional and local services and determine appropriate providers. | - Yes. |
| (32) | F-208: support rural levels of development and not facilitate urbanization. | - Not applicable. |
| (33) | F-209 and F-212: capital facility plans and improvement programs for services to unincorporated King County are consistent with the KCCP. | - At this time CWA’s existing capital facilities for delivery of municipal water supplies consist solely of the Bellevue-Issaquah Pipeline. - CWA also owns the White River Lake Tapps Reservoir Project. - No capital improvements have been identified in relation to its current supplies over the next six years. |
| (34) | F-210: King County helps coordinate development of utility facilities. | - Yes, to the extent applicable, King County will do this. |
| (35) | F-215: King County shall initiate a sub-area planning process with any service provider that declares, in its capital facilities plan, an inability to meet service needs within service area. | - Not applicable. CWA did not identify any inability to meet service needs within its service area. |
| (36) | F-217: where an area-wide sewer, water, or transportation deficiency is identified, King County and applicable service providers shall remedy the deficiency through a joint planning process. | - No area-wide water deficiency is identified. |
| (37) | F-225: King County supports coordination of regional water supply planning, sales of excess water among municipalities, water quality programs, and water conservation and reuse programs. | - Yes, this is done as CWA obtains the majority of its water from the regional system. |
| (38) | F-226: Group A water systems must meet duty to serve requirement within service area as defined under CWSP or by individual water system plans. | - Not applicable. |
| (39) | F-227-231: provides a hierarchy of water supply providers in unincorporated King County, depending on whether within urban growth area or rural areas, with preference for providing water from existing suppliers. | - Not applicable. |
| (40) | F-237: King County supports the use of interties consistent with planning, and implementation of approved ESA and Clean Water Act response requirements. | - CWA members have many interties or meter locations where water is taken from SPU's main transmission lines. |
| (41) | F-239: King County partners with utilities to encourage best management practices and conservation through such means as developing reclaimed water, aggressive water conservation and reuse measures; support planned land uses with reliable service at minimum cost; encourage reclaimed water use, focused on large water users such as golf courses and cemeteries. | - CWA and the County will partner as needed on these issues. - CWA and the County signed a Memorandum of Understanding (MOU) on water resources and supply planning in February 2005. Many planned actions in that MOU have been implemented or completed since 2005. |
| (42) | F-240: Utilities Technical Review Committee (UTRC) to consider (a) consistency with land use plans and development regulations; (b) approved or adopted plans for ground water, ESA, salmon recovery, water resources, watershed planning, regional water supply plan; and (c) the Regional Wastewater Services Plan. | - The UTRC did consider these issues and recommends approval of the Plan. |
| (43) | F-241: in reviewing proposals for modified and expanded service area boundaries, the UTRC must include an evaluation of the utility’s compliance with its comprehensive water system plan, including water conservation elements, and whether it can meet its duty to provide service; no approval of service area where unable to provide service for reasons in RCW 43.20.260. | - CWA is not planning to change its service area. |
| (44) | F-243: public drinking water system reservoirs and watersheds should be managed primarily to protect drinking water supplies, but allow multiple uses when not jeopardizing water quality; downstream uses including recreation, fish, and agricultural resources. | - Not applicable at this time. In the future when CWA uses Lake Tapps as a source this issue will be relevant. |
| (45) | F-244: ground water supplies should be protected by preventing land uses that may adversely affect quantity or quality. | - Not applicable. |