## STAFF REPORT

| Agenda Item: | 8 | Name: | Mike Reed |
| --- | --- | --- | --- |
| Proposed No.: | 2019-B0153 | Date: | October 2, 2019 |

SUBJECT

A briefing to provide a status report on the Conveyance System Improvement Project update.

SUMMARY:

The Conveyance System Improvement Program is updated every five years, as required by the Regional Wastewater Services Plan (RWSP).  In May, 2017, in fulfillment of this requirement, the Wastewater Treatment Division issued a report entitled Conceptual Projects to Meet Identified Capacity Needs.  That report was issued in response to a Regional Needs Assessment prepared in 2015, which identified the current and future capacity needs and constraints in the region’s wastewater conveyance system.  The Conceptual Projects report provided a list of potential projects identified to meet the capacity needs of the wastewater conveyance system.  Today’s briefing will update the Committee on the progress in implementing projects on the project list, and on anticipated projects in upcoming years.

BACKGROUND:

Wastewater that is collected by local utilities and delivered to the regional system, is moved from the point of delivery to regional treatment plants through a network of pipes and pumps that are distributed throughout the region.  This conveyance network includes more than 390 miles of pipeline.

The conveyance system is addressed in the Regional Wastewater Services Plan by a number of policies, including the following:

CP-1:  To protect public health and water quality, King County shall plan, design and construct county wastewater facilities to avoid sanitary sewer overflows.

1.  The twenty-year peak flow storm shall be used as the design standard for the county’s separated wastewater system.

CP-2:  King County shall construct the necessary wastewater conveyance facilities, including, but not limited to pipelines, pumps and regulators, to convey wastewater from component agencies to the treatment plants for treatment and to convey treated effluent to water bodies for discharge.  Conveyance facilities shall be constructed during the planning period of this plan to ensure that all treatment plants can ultimately operate at their rated capacities.

CP-4:  The executive shall update the conveyance system improvement program every five years beginning in 2013 to ensure the program remains current.  The program updates shall provide information on growth patterns, rate of growth and flow projections and report on how this information affects previously identified conveyance needs.  The program updates shall also provide information on changed or new conveyance needs identified since the previous update.

The Conveyance System Improvement Program Plan Update is designed to identify where additional conveyance capacity will be needed through 2060 in the portion of the conveyance system served by separated sewers.  As noted above, the County has adopted a 20-year “peak flow capacity” standard for regional conveyance facilities, in order to prevent sewer overflows.  Facilities must have the capacity to convey peak flows of a magnitude that can be expected to occur during a severe storm event which occurs on average once every 20 years.

The Wastewater Treatment Division (“WTD”) coordinated with the Metropolitan Water Pollution Abatement Advisory Committee (“MWPAAC”) and local sewer utilities to complete the last update.   That process included the following components:

- Update Planning Assumptions

- Complete Regional Conveyance System Needs Assessment

- Develop Conceptual Projects

- Conceptual Project Cost Estimating

- Prioritize Projects.

The Regional Needs Assessment (“RNA”) was completed in 2015.  The RNA identified capacity needs of the region, by regional planning area.  It found that, over the 50-year planning period (2010-2060), 77 conveyance facilities will fall below the 20-year peak flow design standard, or that they are currently below the standard.  Forty of the facilities do not currently meet the standard.

In August 2016, WTD prepared a report on Conceptual Projects to Meet Identified Capacity Needs.  The report described the project options considered for addressing the capacity needs identified above, as follows:

- Paralleling existing conveyance pipes with new pipes

- Upgrading pump stations

- Replacing undersized pipes or pump stations with larger ones

- Diverting flows to other conveyance facilities

- Building storage facilities that reduce peak flow volumes by storing wastewater during high flow periods, until it can be safely conveyed by the downstream system

For facilities that were assessed to have insufficient capacity to convey a 20-year peak flow without surcharging or overflowing under existing conditions, the condition, age and composition of pipes were considered in deciding to whether to parallel or replace them.  Replacement projects were developed for pipes greater than 50 years old and in poor condition.  For relatively new pipes made of durable materials with sufficient capacity available, paralleling projects were recommended.

The size for each new parallel or replacement pipe was determined based on the projected 20-year peak flow for 2060 that would be conveyed through the pipe.  Possible pipeline routes were developed based on GIS data, aerial photographs, and elevations of existing pipelines.  Stream and wetland crossings were avoided if possible; major street crossing were minimized; and public rights-of-way were preferred to private properties.  Flow diversion and storage were considered in those cases where paralleling or replacement were not feasible.

A resulting list of proposed projects and cost estimates was reviewed with local sewer agency representatives, and modified as necessary based on topographic or permitting issues, as well as opportunities to coordinate with planned road and utility projects.

Infiltration/Inflow considerations were to be incorporated into the project development process.  The feasibility of reducing, delaying or eliminating the need for a given project through reduction of infiltration and inflow (known as I/I), as well as opportunities for including reclaimed water conveyance, were to be considered in initial stages of each project.

The map below depicts the Conceptual Conveyance System Improvement project locations included in the May 2017 report.

Following the development of conceptual improvement projects, WTD undertook a prioritization process, to identify high priority projects for early project scheduling.  That prioritization process, in partnership with MWPAAC, utilized criteria such as level of service, operations and maintenance issues, community and local agency input, and coincident benefits, to rank projects.

Today’s briefing is intended to provide the RWQC a progress update on the implementation of the Conceptual Projects report.

ATTACHMENTS

- Estimated Project Costs and Construction Costs for Conceptual Projects

- Conceptual Projects to Meet Identified Capacity Needs Report, May 2017 (link)

http://your.kingcounty.gov/dnrp/library/wastewater/csi/2017-update/1705_CSI-conceptual-projects.pdf

INVITED

- Steve Tolzman, Capital Projects Manager, Wastewater Treatment Division, Department of Natural Resources and Parks

https://your.kingcounty.gov/dnrp/library/wastewater/csi/2017-update/1705_CSI-conceptual-projects.pdf

Whether to build a second ‘parallel’ pipe or replace the existing pipe